EUROPEAN SOUTHERN OBSERVATORY

Organisation Européenne pour des Recherches Astronomiques dans l'Hémisphère Austral

Europäische Organisation für astronomische Forschung in der südlichen Hemisphäre

- VLT PROGRAMME

- VERY LARGE TELESCOPE

- LCU Common Software

---

- LCU Server Framework

User Manual

- Doc. No.: VLT-MAN-ESO-17210-2252

- Issue: 1.0

Date: 2000-10-25

Prepared: P.Duhoux				2000-10-25

Name				Date		Signature

- Approved:  B. Gustafsson

Name				Date		Signature

- Released:  G. Raffi

Name				Date		Signature

- VLT PROGRAMME * TELEPHONE: (089) 3 20 06-0 * FAX: (089) 3 20 06 514

- CHANGE RECORD

| ISSUE | DATE | SECTION/PAGE AFFECTED | REASON/INITIATION DOCUMENTS/REMARKS |
| --- | --- | --- | --- |
| 1.0 | 2000-10-25 | All | First preparation |

- TABLE OF CONTENTS

1	Introduction	5

1.1	Purpose & Scope	5

1.2	Applicable Documents	5

1.3	Reference Documents	6

1.4	Acronyms	7

1.5	Stylistic Conventions	7

1.5.1	Text	7

1.5.2	Diagrams	7

2	User Manual	8

2.1	Introduction	8

2.2	LCU Server Framework	8

2.2.1	lsf: the LCU Server Framework	8

2.2.2	lsftpl : the LCU Server Application Template	13

2.3	LCU Server Application	15

2.3.1	Module Creation	15

2.3.2	Module Configuration	15

2.3.3	Specific Implementation	18

2.3.4	Scan Links	19

2.3.5	Database Configuration	19

2.3.6	Default panel & UIF widget	20

2.3.7	Application Makefiles	20

2.3.8	Implementation Rules for the Command Handlers	21

2.3.9	Hook functions	21

2.3.10	Miscellaneous	24

3	Reference	25

3.1	Utilities	25

3.1.1	lsf(1)	25

3.1.2	lsfCreate(1)	30

3.1.3	lsfConfig(1)	31

3.1.4	lsfBackup(1)	33

3.2	Code	34

3.2.1	lsfServer(3)	34

3.2.2	lsfStandard(3)	39

3.2.3	lsfSignalHandlers(3)	41

3.2.4	lsfControl(3)	42

3.2.5	lsfDevice(3)	45

3.2.6	lsfSignal(3)	47

3.2.7	lsfMotor(3)	50

3.2.8	lsfSerial(3)	53

3.2.9	lsfTaskDev(3)	56

3.2.10	lsfSoftDev(3)	59

3.3	Database Classes	62

3.3.1	lsfTemplate.db	62

3.3.2	lsfDB_SERVER(5)	64

3.3.3	lsfDB_CONTROL(5)	65

3.3.4	lsfDB_DATA(5)	66

3.3.5	lsfDB_DEVICE(5)	67

3.3.6	lsfDB_SIGNAL(5)	68

3.3.7	lsfDB_MOTOR(5)	70

3.3.8	lsfDB_SERIAL(5)	71

3.3.9	lsfDB_TASKDEV(5)	72

3.3.10	lsfDB_SOFTDEV(5)	73

3.4	Include Files	74

3.4.1	lsfDefines.h	74

4	Installation Guide	77

4.1.1	WS Environment	77

4.1.2	LCU Environment	77

4.1.3	Scan Links	79

4.1.4	Verification	80

4.1.5	Database configuration	80

<!DOCTYPE HTML PUBLIC "-//IETF//DTD HTML//EN">

- Introduction

  - Purpose & Scope

This document describes the usage of the LCU Server Framework lsf from a programmatic point of view. It is intended to provide all the necessary information for an application developer to create, configure, customize and install LCU Server applications. It describes all the necessary steps that must be performed for the implementation of a new Software Device.

The intended readers are application developers of VLT Software who need to develop, test or maintain LCU Server Applications, based on this framework.

The author assumes the reader has a good knowledge of the VLT Standards and Software Engineering practices, especially experience with Real-Time and Object Oriented concepts and UML Notation would be an asset.

Beside the Introduction, the document is built in 3 parts:

- the User Manual describes the concepts and usage of the LCU Server Framework

- the Reference section provides the necessary man-pages for all the available scripts, commands and code

- the Installation Guide explains how to install a new LCU Server Application in the VLT environments

<!-- Standard footer -->

<!-- hhmts start -->Last modified: Thu Oct 26 12:50:45 METDST 2000 <!-- hhmts end -->

  - Applicable Documents

The following documents, of the exact issue shown, form part of this document to the extend specified herein. In the event of conflict between the documents referenced herein and the content of this issue, the content of this document shall be considered as a superseding requirement.

| Reference | Document Number | Issue | Date | Title |
| --- | --- | --- | --- | --- |
| [AD 01] | VLT-SPE-ESO-10000-0011 | 2.0 | 30/09/1992 | VLT Software Requirements Specifications |
| [AD 02] | VLT-PRO-ESO-10000-0228 | 1.0 | 10/03/1993 | VLT Software Programming Standards |
| [AD 05] | VLT-MAN-ESO-17200-0888 | 1.0 | 17/08/1995 | VLT Common Software Overview |
| [AD 06] | VLT-MAN-ESO-17210-0642 | 1.11 | 22/11/1999 | VLT Common Software Installation Manual |
| [AD 07] | VLT-MAN-ESO-17210-0855 | 3.0 | 30/10/1998 | VLT Software Environments Common Configuration User Manual |
| [AD 08] | VLT-MAN-ESO-17210-0667 | 1.0 | 03/12/1996 | Guidelines for the Development of VLT Application Software |
| [AD 09] | VLT-SPE-ESO-17210-2051 | 1.0 | 2000-03-10 | LCU Common Software - LCU Server Framework Detailed Design |

  - Reference Documents

The reference documents contain background information required to fully understand the structure of this document, the terminology used, the software environment in which the product shall be integrated and the interface characteristics to the external systems.

VLT Project Software documents are available from the ESO VLT Project Archive or online at the following URL:
http://www.eso.org/projects/vlt/sw-dev/ftp/index.html

| Reference | Document Number | Issue | Date | Title |
| --- | --- | --- | --- | --- |
| [RDV 01] | VLT-MAN-ESO-17210-0619 | 1.6 | 20/10/1999 | VLT Central Control Software - User Manual |
| [RDV 02] | VLT-MAN-SBI-17210-0001 | 3.5 | 20/10/1999 | LCU Common Software - User Manual |
| [RDV 03] | VLT-MAN-ESO-17210-0707 | 1.4 | 20/10/1999 | CCS On-Line Database Loader - User Manual |
| [RDV 04] | VLT-SPE-ESO-17230-0819 | 1.3 | 18/09/1997 | Mode Switching - Design Description |
| [RDV 05] | VLT-MAN-ESO-17210-0600 | 1.7 | 02/10/1998 | LCC Motor Control Module - User Manual |

The following non-VLT documents are referenced in the document:

| Reference | Title | Authors | Publisher, Date |
| --- | --- | --- | --- |
| [RD 01] | IEEE Std 1012, Standard for Software Verification and Validation Plans |  | 1986 |
| [RD 02] | IEEE Std 830, Recommended Practice for Software Requirements Specifications |  | 1993 |
| [RD 03] | IEE (UK), Guidelines for the Documentation of Software in Industrial Computer Systems |  | 1985 |
| [RD 04] | The Unified Modeling Language Reference Manual | J. Rumbaugh, G. Booch, I. Jacobson | Addison-Wesley, 1998 |
| [RD 05] | The Unified Modeling Language User Guide | G. Booch, J. Rumbaugh, I. Jacobson | Addison-Wesley, 1998 |
| [RD 06] | The Unified Software Development Process | I. Jacobson, G. Booch, J. Rumbaugh | Addison-Wesley, 1999 |
| [RD 07] | Real-Time UML - Developing Efficient Objects For Embedded Systems | B. Douglas | Addison-Wesley, 1998 |
| [RD 08] | Design Patterns - Elements of Reusable Object-Oriented Software | E. Gamma, R. Helm, R. Johnson and J. Vlissides | Addison-Wesley, 1994 |

  - Acronyms

This document employs several abbreviations and acronyms to refer concisely to an item, after it has been introduced. The following list is aimed to help the reader in recalling the extended meaning of each short expression:

| [AD xx] | Applicable Document #xx |
| --- | --- |
| CCS | Central Common Software |
| DB | Database |
| ESO | European Southern Observatory |
| I/O | Input/Output |
| LAN | Local Area Network |
| LCC | LCU Common Software |
| LCU | Local Control Unit |
| MCM | Motor Control Module (part of LCC) |
| OLDB | On-line Database |
| OO | Object Oriented |
| [RD xx] | Reference Document #xx |
| RT | Real-Time |
| TIM | Time Interface Module |
| UC | Use Case |
| UML | Unified Modeling Language |
| VME | Versa Module Eurocard |
| WS | (Unix) Work Station |

  - Stylistic Conventions

    - Text

The following styles are used:

- bold : in the text, for commands, filenames, pre- and suffixes as they have to be typed.

- italic : in the text, for parts that have to be substituted with the real content before typing.

- teletype : for examples.

- <name> : in the examples, for parts that have to be substituted with the real content before typing.

    - Diagrams

All diagrams are based on the UML notation, as described in [RD 04] and given in Appendix.
Besides the classical class notation, three dedicated icons are used throughout the class diagrams for representing the UML standard class stereotypes ([RD 06] pp. 183ff) and shown below:

- <<boundary>>:  model interactions between the system and its actors.

- <<entity>>:  model information and associated behavior of phenomenon, concept such as a real-life objects; often persistent.

- <<control>>:  represent coordination, sequencing, transactions and control of other objects; often used to encapsulate the control related to a use-case.

<!-- Standard footer -->

<!-- hhmts start -->Last modified: Thu Oct 26 14:15:25 METDST 2000 <!-- hhmts end -->

<!DOCTYPE HTML PUBLIC "-//IETF//DTD HTML//EN">

- User Manual

  - Introduction

In this chapter, the concept of the LCU Server Framework will be described. This section is followed by the practical description of its usage through the whole creation and customization process of a new LCU Server Application.

Naming Conventions: In this chapter, the following naming conventions apply: the generic names devtype  and Devtype  differ only by the case of the first letter (lower/upper). The first case indicates the name of the device type, the latter is the name of the device type when concatenated with the module prefix as in appDevtype.

  - LCU Server Framework

The intent, motivation and consequences of the framework have been discussed in detail in [AD 09]. The framework implements the minimum behavior for all the Standard Commands and offers the expected extensibility, flexibility and changeability while defining a fixed architecture and improving the overall maintainability as most of the code is provided and reused.

    - lsf: the LCU Server Framework

The framework consists in a collection of reusable components that implement the standard behaviour for a number of devices. It provides a unique and fully re-entrant set of common procedures that need not be duplicated by new applications.

As of this issue, lsf supports the following devices:

- I/O signals: digital and analog signals based resp. on the VLT standard Acromag AVME 948x Digital I/O and VMIVME-3111 Analog I/O VME boards

- motors: any kind of motor as supported by MCM

- serial communication links: based on the VxWorks tyCo driver and on the VLT standard esd ISER8 Serial Interface Board.

- tasks: any periodic task running in background as part of the Software Device

- software devices: any lsf-based sub-Software Device running on the same LCU.

Architecture: The purpose of a framework is to impose the architecture of the applications based on and to provide the implementation of the standard behaviour. The application is consequently responsible for implementing the specialized functionality as of its design.

The architecture is hierarchical: The server object lsfServer is responsible for handling the application global data and hosts the commands of the application; these commands might involve no device (pure software command) or devices of various types (combined command). The actual control of the devices is performed from within the object lsfControl that dispatches the standard commands to the respective devices. Finally for each device type, an object lsfDeviceType implements the commands of this type of device.

Two additional objects complete this view: the task lsfInitAll that creates the application at boot time, and the task lsfMonitor, created at boot time, that monitors periodically the state and sub-state of all the devices and updates consequently the state and sub-state of the application.

Devices: Each device is characterized by its type as listed above. All devices of one type are supposed to behave in a similar manner in response to the standard commands. Consequently all the methods associated to one device type perform the same actions on all the devices of this type.

Standard Behaviour: The standard commands are implemented within the framework. They define the standard behaviour of each standard command for each type of device. The standard behaviour is edicted in [RDV 02] for Software Devices and State management. The standard behaviour can not be modified but only extended by means of dedicated hooks functions, which are application specific and implement additional actions required by the application design (e.g. set a particular I/O signal prior to go Online).

States and Sub-States: Each device is characterized by its state and sub-state. The state is defined by LCC ([RDV 02]) and takes the values OFF, LOADED, STANDBY or ONLINE.
Each group of devices of a type is as well characterized by a state and sub-state value; the state is set as being the lowest state of all the devices of this group.
The global application state is then the lowest state value of all the devices.

The sub-state is used to indicate the detailed activity of a device, or group of devices; the following sub-states have been introduced:

- Idle: the device is inactive

- Error: an error has occured

- Timeout: the requested action has timed out

- Initializing: the device is performing the initialization procedure

- Monitoring: monitoring is active in background

- Moving: for motors only when performing a motion

- Active: any other activity like e.g. Waiting for an event, in a Control Loop etc...

The 2 sub-states Error and Timeout are transcient states that reset to Idle on recovery, or on next request.

The global sub-state of the software device is set to:

- Idle: when all the devices are inactive

- Error: if one device went in error.

- Timeout: if one device timed out.

- Initializing: if one device is initializing

- Active: if one device or more is active

Database: All relevant configuration and runtime data is stored in the OLDB of the LCU, which is partly mirrored on the WS so as to enable the use of the scan system.

Each type of device is mapped in the database by a dedicated class. All the device classes are derived from the base abstract class lsfDB_DEVICE as depicted in the class hierarchy:

Note that a Software Device is a sub-class of lsfDB_SOFTDEV, thus allowing recursive construction: a Software Device can be a sub-Software Device of another one.

The structure of the database is fixed as illustrated in the diagram below:

Note the multiplicity of the various instances as a means to scale the database to the exact needs of the application.

Application Data: The class lsfDB_DATA is an abstract class. It does not contain any attribute as it is intended to be specialized (by sub-classing) to the needs of the application. This class has been introduced so as to allow the application to store data that shows following characteristics at a known location:

- the data is not strictly device dependent (e.g. temperatures, or telemetry values), or shall be hardware independent (i.e. should a thermometer device be exchanged from a device with digital interface to a one with serial interface)

- the data might be accessed from within another device (e.g. a background task) for further processing

- the data might need to be scanned to the WS independently on the devices producing it.

    - lsftpl : the LCU Server Application Template

The module lsftpl is a template application based on lsf that provides an implementation template for all available device types. In particular, it gives examples of ACI and API functions for signals and motors for addressing one or all devices of this type.

The template implements the control of 16 devices: 8 I/O signals (4 analogue and 4 digital where from 2 input and 2 output signals), 2 motors, 3 serial communication links (2 using the ISER 8 board, and one /tyCo device), 2 sub-ordinated software devices (swdtst1 and swdtst2) and 1 task device performing periodic monitoring) as illustrated in the class diagram below:

It implements the functionality as shown in the Use Case diagram:

N.B.: as of this version, both the encoder and network devices are not yet supported.

The following section describes all the steps necessary to create a new application from this template.

  - LCU Server Application

This section describes how to create a LCU Server Application based on the Framework.

It assumes the detailed analysis of the package is complete: all interacting devices, and commands and interfaces have been identified and described.

    - Module Creation

The creation of a new LCU Server Application app from the template is made by means of the utility lsfCreate(1). It makes a copy of the template module lsftpl in its last version available under CMM, and performs all string replacements: all occurences of lsftpl are substituted by app, as well the occurences of LSFTPL are substituted by APP.

Example: 
<DIV>
lsfCreate app 
Retrieving module 'lsftpl' ... done 
Renaming module 'lsftpl' to 'app' ... done 
Renaming all files with prefix 'app' ... done 
Replacing pattern 'lsftpl' by 'app' ... done 
1.24u 49.41s 1:59.60 42.3% 
</DIV>

The module app is ready. It needs now be configured for the devices and commands.

    - Module Configuration

The configuration of the application is kept central in the file app/ws/config/app.cfg. This file must be edited to the needs of the application. The module configuration contains 3 parts:

- module:
LSF.MODULE.NAME app

- devices:
LSF.DEVICE.NUM number of devices D
For each device d (from 1 to D)
LSF.DEVICEd.NAME: name of the device (19 char. max), unique for the application.
LSF.DEVICEd.TYPE: type of the device:

- analog,  digital: I/O signal

- motor: motorized axis

- serial: communication link

- taskdev: task

- softdev: sub-Software Device (another LSF-based application to which this application will communicate via the Message Server)

LSF.DEVICEd.CLASS: when applicable the name of the specialized database class appDB_devtype  as derived from the corresponding generic device class lsfDB_devtype. For the motors, the motor specialized class name (motDVAMI etc... see [RDV 05])
LSF.DEVICEd.PARAM: for motors only the dimensions of database tables for the named positions (P), named speeds (V) and initialization sequence (I), following the syntax:  P p,V v,I i.

- commands:
LSF.COMMAND.NUM number of commands C
For each command c (from 1 to C)
LSF.COMMANDc.NAME: name of the command following the naming recommendations stated in [RDV 02]
LSF.COMMANDc.DEST: device type for which the command is intended (see the list of device types above); in addition, the destination may be set to server for any command which either does not involve any device, or commands that involve many devices of various types
LSF.COMMANDc.GROUP: as described in [RDV 02], the associated command group (PUBLIC,  MAINTENANCE or TEST)
LSF.COMMANDc.ENTRY: the name of the entry function associated to the command and to be declared in the Command Interpreter Table (CIT). The ACI function name must follow the convention: Iapp<Devtype><Entry>, where Devtype  is the device type as declared under LSF.COMMANDc.DEST and Entry  is the name entered under this key-word
LSF.COMMANDc.INVOKE: the invocation mode FUNCTION or TASK
LSF.COMMANDc.OPTION: for the invocation mode TASK only, the task options following the syntax: <taskName>,[<priority>], [<task flags>],[<stack size>][,<options>]. 
where the task name shall be built as tapp<command name> 
Note: The option REGISTER must be omitted since it is handled internally (all tasks are registered to allow CCS communications)

Example:

#*****************************************************************************

# E.S.O. - VLT project

#

# "@(#) $Id: app.cfg,v 3.4 2000/07/21 09:22:08 vltsccm Exp $"

#

# who       when        what

# --------  ----------  ----------------------------------------------

# pduhoux   2000-04-07  created

#

#

# MODULE

#

LSF.MODULE.NAME     app

#

# DEVICES

#

LSF.DEVICE.NUM      1

LSF.DEVICE1.NAME    NDF

LSF.DEVICE1.TYPE    motor

LSF.DEVICE1.CLASS   motDVAMI

LSF.DEVICE1.PARAM   P 7,V 3,I 6

#

# COMMANDS

#

LSF.COMMAND.NUM       1

LSF.COMMAND1.NAME     SETNDF

LSF.COMMAND1.DEST     motor

LSF.COMMAND1.GROUP    PUBLIC

LSF.COMMAND1.ENTRY    SetFilter

LSF.COMMAND1.INVOKE   TASK

LSF.COMMAND1.OPTION   appSETNDF,,,40000

# ___oOo___

The effective configuration of the LCU Server Application app is made by means of the utility lsfConfig(1).

It performs following steps:

- Parse the configuration file app/ws/config/app.cfg and display all the devices by type and the commands by group

- Generate the include file app/lcu/include/appDeviceList.h containing the macro definition of all devices

- Generate the database file app/lcu/dbl/app.db for the instantiation of the database point.

- Generate the makefile app/lcu/src/Makefile

- Generate the CDT partial files app/lcu/src/appPublic.cdt, app/lcu/src/appMaintenance.cdt and app/lcu/src/appTest.cdt. The files are created only if at least one command has been declared in this group.

- Generate the CIT partial files app/lcu/src/app<Devtype>.cit for each referenced device type Devtype.

- Move all the unused files to app/lcu/tmp
(The unused files are the files associated to the control of the device types not referenced in the configuration file. They are moved to this directory prior to be removed by the user when preparing for archive.)

- Generate the log file app/ChangeLog and add an entry for the configuration sequence.

Important Note: The utility lsfConfig shall be called once as its actions are destructive for some files. Any application dependent modification made to the partial CDT files will be lost during this process, as these files are re-created. A posteriori addition of a device of a new type is not supported.

Example:

> lsfConfig app

Parse file 'app/ws/config/app.cfg' ... done

Found 1 device:

1 Motor    : NDF

Found 1 PUBLIC command:

1 Motor    : SETNDF

Found 0 MAINTENANCE command

Found 0 TEST command

Generate 'app/ws/config/app.dbcfg' ... done

Generate 'app/lcu/include/appDeviceList.h' ... done

Generate 'app/lcu/dbl/app.db' ... done

Generate 'app/lcu/src/Makefile' ... done

Generate 'app/lcu/src/appPublic.cdt' ... done

Generate 'app/lcu/src/appMotor.cit' ... done

Move unused files to ./lcu/tmp ... done

Add change log entry ... done

>>> Remember to edit the CDT files ...

Module configuration for 'app' ... done"

>

    - Specific Implementation

After the module has been tailored to the needs of the application, some files need be edited to implement the behaviour and specify the interfaces as of the design.

- app/lcu/src/appPublic.cdt,
app/lcu/src/appMaintenance.cdt,
app/lcu/src/appTest.cdt: For each command the parameters and replies must be described.

- app/lcu/dbl/appDB_DEVICE.class: Each application specific device class must be defined as a sub-class of the corresponding device type lsfDB_DEVTYPE

- app/lcu/dbl/appDB_DATA.class:  If any extra data storage is needed in the database by the application, the class appDB_DATA must be described in this file

- app/lcu/include/app<Devtype>.h:  for each referenced device type, the prototypes of the API functions associated to the commands must be declared. The file app/lcu/include/appServer.h is also subject to modification.
The API function shall be named following the convention for the ACI function names without the leading I.

- app/lcu/src/app<Devtype>.c:  Each API function must be implemented in the respective file.

- app/lcu/src/app<Devtype>Interface.c:  Each ACI function must be implemented in the respective file.

For the sake of clarity, one might group the API functions dedicated to one specific device into one separate file. These files may be included into the device type associated file.

The class diagram shown below depicts the device control structure for a specific command:

    - Scan Links

For application implementing the WS counterpart of the database, the scan links must be configured. These links are described in the file app/ws/config/app.scan.

    - Database Configuration

At boot-time, the database branch of the module need be configured. The configuration is kept in the file app/ws/config/app.dbcfg and restored from $VLTROOT/config, resp. from $INTROOT/config at startup. This file is mandatory for starting up the server, however the database is then solely basically configured and might not permit to initialize the system. By means of the dedicated engineering tools, the database can be properly configured. Once the initialization is successful, the utility lsfBackup(1) creates a file ./app.dbcfg that contains the relevant database configuration.

Example:

> cd app/ws/config

> lsfBackup -e lcuEnv -m app

Generating input file './app.inp' ... done

Performing database backup into  './app.dbcfg' ... done

>

The file ./app.inp is a temporary file that contains the list of database points and attributes to backup. At backup completion, it is removed.

    - Default panel & UIF widget

A default panel appgui is provided by the template that instantiates the UIF widget lsfState_uifClass. This widget shows the associated LCU environment name, a button for sending standard commands, a STOP button, and the status information of the module (state, subState and operational mode).

In addition, the Command Feedback Window shows the last 2 replies.

    - Application Makefiles

The WS part is built as described in app/ws/src/Makefile. It makes the panel app/ws/src/appgui.pan and installs the two configuration files app/ws/config/app.scan and app/ws/config/app.dbcfg under $XXXROOT/config.

Example:

> cd app/ws/src

> make all install

...

. . . 'all' done

...

. . . installation done

>

The LCU part is built as described in app/lcu/src/Makefile.

It invokes 3 utilities prior to building the code (target "all"):

- lsfMakeCIT: merge all the partial CIT files app/lcu/src/app*.cit and the common CIT file $XXXROOT/vw/CIT/lsfCommon.cit into app/lcu/CIT/appServer.cit;

- lsfMakeCDT: merge all the partial CDT files app/lcu/src/app*.cdt and the common CDT files $XXXROOT/CDT/lsf*.cdt into app/lcu/CDT/appServer.cdt;

- lsfMakeINC: generate all the files app/lcu/include/app*Interface.h from the partial CIT files app/lcu/src/app*.cit.

Example:

> cd app/lcu/src

> make all

. . . 'CIT' done

. . . 'CDT' done

Include file '../include/appMotorInterface.h' done

. . . 'include files' done

...

== Building executable: ../bin/app

. . . 'all' done

>

Note:  The final CDT and CIT files are deleted by the Makefile. Remember to perform the modifications in the ./src directory.

    - Implementation Rules for the Command Handlers

As stated in the rules applying to a Software Device, the behaviour of each command is implemented in a Command Handler. This handler must show the 2 ACI and API interfaces:

- ACI function: the ACI function name is built on the following scheme:

ccsCOMPL_STAT Iapp<Devtype><Entry>

( ... , ccsERROR *error )

It is responsible for parsing the command parameters, invoking the API function and building the reply buffer as defined in the CDT entry of this command.

- API function: the API function name is built on the following scheme:

ccsCOMPL_STAT app<Devtype><Entry>

( ... , ccsERROR *error )

It is the core implementation of the behaviour of the command. It must be callable from another application. It receives application specific parameters and the error stack pointer, and returns the completion status.

    - Hook functions

A hook function is implementing an additional behaviour. Four kinds of hooks are supported:

- Constructors/Destructors: One application specific constructor and one application specific destructor per device name, resp. device type may be implemented. These functions will be invoked according to the following scheme:

- At boot time, the object constructors will be invoked as depicted in the following Sequence Diagram.

The application specific constructors have the prototype:

ccsCOMPL_STAT app<devType>Constructor ( IN  void       *devData,

IN  const char *devName,

OUT ccsERROR   *error )

ccsCOMPL_STAT app<devName>Constructor ( IN  void       *devData,

IN  const char *devName,

OUT ccsERROR   *error )

- At exit time, the object destructors will be invoked as depicted in the following Sequence Diagram.

The application specific destructors have the prototype:

ccsCOMPL_STAT app<devType>Destructor  ( IN  void *devData )

ccsCOMPL_STAT app<devName>Destructor  ( IN  void *devData )

- Standard Commands: 3 hooks per standard command may be implemented. The hook function names must follow the prototype convention:

ccsCOMPL_STAT appStd<Command><Type>Hook ( IN  lsfSERVER_DATA *serverData,

OUT ccsERROR       *error )

where Command is the name of the command (Init, Standby, Online, Stop, Off, Exit, Selftest, Test),
and Type is the hook type (Pro, Mid, Epi) resp. for the prologue, the intermediate and the epilogue additional statements. The intermediate hook is invoked only together with at least one sub-Software Device and one Hardware Device.

The hooks are located in the file appStdHooks.c. They do not need any prototype declaration.

- Signals Initialization hook: a dedicated hook might be implemented that allows the proper setting of the signals at initialization, thus setting the sub-system in a known state. The hook function has the following prototype:

ccsCOMPL_STAT appSignalInitHook ( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

OUT ccsERROR       *error )

- Task device hooks: One hook function per task device must be implemented, additional 2 hooks are optional.
The mandatory hook function implements the core of the task and the prototype is:

ccsCOMPL_STAT app<TaskDevName> ( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

OUT ccsERROR        *error )

where TaskDevName is the name of the task device (starting with upper case).
The 2 additional hooks are invoked whenever implemented before entering the loop, resp. when terminating. The hook functions must be prototyped:

ccsCOMPL_STAT app<TaskDevName>ProHook ( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

OUT ccsERROR        *error )

resp.

ccsCOMPL_STAT app<TaskDevName>EpiHook ( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

OUT ccsERROR        *error )

    - Miscellaneous

- Adding source files: There is no limitation in adding new source or include files to the module repository. These files must be listed in the makefile either under the list app_OBJECTS for a source file, or under INCLUDES for an include file to be installed, see also Makefile(5).

<!--

<H3>Summary</H3>

<UL>

<LI><B><U>:</U></B>&nbsp;

-->

<!-- Standard footer -->

<!-- hhmts start -->Last modified: Thu Oct 26 14:23:30 METDST 2000 <!-- hhmts end -->

<!DOCTYPE HTML PUBLIC "-//IETF//DTD HTML//EN">

- Reference

In this section, the man-pages of the utilities, objects anf database classes are provided. The public include files are appended.

| Utility | Code | Database Class | Include File |
| --- | --- | --- | --- |
| lsf(1) |  | lsfTemplate.db | lsfDefines.h |
| lsfCreate(1) |  |  |  |
| lsfConfig(1) |  |  |  |
| lsfBackup(1) |  |  |  |
|  | lsfServer(3) | lsfDB_SERVER(5) | lsfServer.h |
|  | lsfStandard(3) |  |  |
|  | lsfHandleBreak(3) lsfHandleKill(3) |  |  |
|  | lsfControl(3) | lsfDB_CONTROL(5) lsfDB_DATA(5) | lsfControl.h |
|  | lsfDevice(3) | lsfDB_DEVICE(5) | lsfDevice.h |
|  | lsfSignal(3) | lsfDB_SIGNAL(5) | lsfSignal.h |
|  | lsfMotor(3) | lsfDB_MOTOR(5) | lsfMotor.h |
|  | lsfSerial(3) | lsfDB_SERIAL(5) | lsfSerial.h |
|  | lsfTaskDev(3) | lsfDB_TASKDEV(5) | lsfTaskDev.h |
|  | lsfSoftDev(3) | lsfDB_SOFTDEV(5) | lsfSoftDev.h |

<!-- Standard footer -->

<!-- hhmts start -->Last modified: Fri Aug 11 08:21:18 METDST 2000 <!-- hhmts end -->

  - Utilities

    - lsf(1)

NAME

lsf - LCU Server Framework

SYNOPSIS

> lsfCreate <modName>

> emacs <modName>/ws/config/<modName>.cfg

> lsfConfig <modName>

> emacs <modName>/lcu/src/<modName>Public.cdt

> cd <modName>/ws/src  ; make clean all man install ; cd -

> cd <modName>/lcu/src ; make clean all man install ; cd -

DESCRIPTION

lsf is a framework for developing LCU Server Applications. It

consists of a WS part dedicated to the generation and

configuration/customization  of the target application module and

of a LCU part providing the application architecture and implementing

the minimum behaviour for the standard commands.

1 - The WS part:

1.1 Script lsfCreate(1): creates a software module compliant with

the VLT Standards from a template.

1.2 Script lsfConfig(1): customizes the software module according

to the software device description file.

1.3 The description file: is an ASCII file containing the definition

of all devices (hardware and software) to be controlled by the

application and of all the commands to be accepted by the

application.

The devices are grouped in 7 types:

- signal (analog or digital) based on VLT AIO resp. ACRO boards

- motor (all types as supported by MCM)

- serial communication link (RS232, RS422 or RS485) based on

VxWorks tyCo driver or VLT ISER8 board.

- software device (any lsf-based application to be controlled

from this software device)

- tasks (any periodic background process)

- encoder [not supported yet]

- network [not supported yet]

The commands are grouped in 3 types:

- public for normal operations

- maintenance for software/hardware verification work

- test with restricted access to technical staff

1.4 The database is a partial mirror (based on the scan system) of

the LCU database

2 - The LCU part:

2.1 Code: the binary code lsf is to be downloaded to the LCU prior

to loading the application software. It implements the minimum

behaviour of the supported devices for the standard commands.

2.2 Database: the database architecture is provided.

The device classes may be customized by sub-classing

to the needs of the application.

3 - Creating a new application

The creation of a new module is made by the utility lsfCreate.

It receives the name of the module <mod> as argument.

The module name shall not be registered in the CMM archive.

Example:

From the parent directory of <mod>

> lsfCreate mod

Retrieving module 'lsftpl' ... done

Renaming module 'lsftpl' to 'mod' ... done

Renaming all files with prefix 'mod' ... done

Replacing pattern 'lsftpl' by 'mod' ... done

1.24u 49.41s 1:59.60 42.3%

>

4 - Customizing the software device

The customizatin of a new module is made by the utility lsfConfig.

The file <mod>/ws/config/<mod>.cfg must be edited and its content

modified to the needs of the application <mod>.

LSF.MODULE.NAME     <mod>

LSF.DEVICE.NUM      << the number of devices to be controlled >>

Mandatory for each device <d>:

LSF.DEVICEd.NAME    << the name of the device,

it defines the name of a database point >>

LSF.DEVICEd.TYPE    << the type of the device:

analog, digital: for signals

serial: for serial communication links

motor: for motorized axis

taskdev: for a periodic background process

softdev: for a sub-software device,

it defines the type of control, and the

associated database class >>

The following 2 entries are optional:

LSF.DEVICEd.CLASS   << the specialized database class name mapping

the device. For motors, the motor class

name (e.g. motDVAMI) >>

LSF.DEVICEd.PARAM   << for motors only, the number of entries in

the named position (P), named speed (V),

initialization sequence (I) and unit conversion

methods (U), following the syntax:

"P <p>,V <v>,I <i>,U <u>" >>

LSF.COMMAND.NUM     << the number of commands to be accepted,

in addition to the standard commands >>

Mandatory for each command <c>:

LSF.COMMANDc.NAME   << the name of the command as specified for

the CDT (7 char.) >>

LSF.COMMANDc.DEST   << signal, motor, serial, softdev or server

where "server" indicates that the command

is to be processed internally (no device

interaction).

NB: task devices do not process commands >>

LSF.COMMANDc.GROUP  PUBLIC, MAINTENANCE or TEST

LSF.COMMANDc.ENTRY  << API Funtion name <Fct> following the rule:

<mod><Devtype><Fct>()

The ACI function name is built automatically

as I<mod><Devtype><Fct>() in the corresponding

CIT file. Where <Devtype> is the destination

device type (e.g. "Signal") >>

LSF.COMMANDc.INVOKE TASK or FUNCTION

LSF.COMMANDc.OPTION << for TASK invocation only, the parameters

as for the CIT entries without REGISTER >>

Example:

From the parent directory of <mod>

> lsfConfig mod

Parse file 'mod/ws/config/mod.cfg' ... done

Found 11 devices:

2 Motor    : M1 M2

8 Signal   : AI1 AO3 AO4 AI8 DO1 DI2 DI3 DO4

1 Taskdev  : bgTask

Found 6 PUBLIC commands:

1 Motor    : MOVEA

4 Signal   : RASIG WASIG RDSIG WDSIG

1 Server   : MAINCMD

Found 0 MAINTENANCE command

Found 0 TEST command

Generate 'mod/ws/config/mod.dbcfg' ... done

Generate 'mod/lcu/include/modDeviceList.h' ... done

Generate 'mod/lcu/dbl/mod.db' ... done

Generate 'mod/lcu/src/Makefile' ... done

Generate 'mod/lcu/src/modPublic.cdt' ... done

Generate 'mod/lcu/src/modServer.cit' ... done

Generate 'mod/lcu/src/modMotor.cit' ... done

Generate 'mod/lcu/src/modSignal.cit' ... done

Move unused files to ./lcu/tmp ... done

Add change log entry ... done

>>> Remember to edit the CDT files ...

Module configuration for 'mod' ... done

>

5 - Application specific code

The following files must be edited:

- <mod>/lcu/src/<mod>*.cdt: describe all the command parameters

and reply

- <mod>/lcu/dbl/<mod>DB_DEVICE.class: define the sub-classes

of lsfDB_DEVICE (or derived) which have been referenced in the

device description file <mod>/ws/config/<mod>.cfg

- <mod>/lcu/src/<mod><Devtype>.c: for each referenced device type

<devtype>, implement the API functions

- <mod>/lcu/include/<mod><Devtype>.h: declare the API function

prototypes

- <mod>/lcu/src/<mod><Devtype>Interface.c: for each command,  in

implement the ACI functions

6a - Extend the behaviour of the standard commands

For each of the standard commands, up to 3 hook functions might be

defined following the prototype:

ccsCOMPL_STAT <mod>Std<cmd><typ>Hook

( IN  lsfCONTROL_DATA *controlData,

OUT ccsERROR        *error)

where

<mod> is the module name,

<cmd> is the standard command name (Init, Online etc...)

<typ> is the hook index (Pro,Mid,Epi)

Pro: hook is invoked when entering the method,

Mid: hook is invoked between Software and Hardware device control

Epi: hook is invoked before leaving the method.

The first (Pro) and last (Epi) are called from within the standard

object regardless on the number of devices. The middle hook (Mid) is

only invoked from within this object.

The prototypes to these functions do not need be declared. The hook

functions shall be located in <mod>/lcu/src/<mod>Standard.c

In addition, for the signals only the optional hook function

ccsCOMPL_STAT <mod>SignalInitHook

( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

OUT ccsERROR       *error )

might be implemented in <mod>/lcu/src/<mod>Signal.c

6b - Extend the behaviour of the device constructors/destructors

For each device type and each device, a specific additional constructor

resp. destructor might be implemented. These functions must be defined

following the prototype:

ccsCOMPL_STAT <mod><devName>Constructor ( IN  void       *devData,

IN  const char *devName,

OUT ccsERROR   *error)

void          <mod><devName>Destructor  ( IN  void       *devData )

The user specific constructors are invoked at completion of the

standard constructor for each device type, then for each device of

this type;

the user specific destructors for each device of each type followed

by theuser specific destructor for this device type are invoked

prior to the standard destructor.

7 - Task devices

The task devices are scheduled according to the state of the

Software Device and is configurable in the database.

The periodic loop associated to a task device is implemented within

the framework. The core function of the task is mandatory and must

be prototyped:

ccsCOMPL_STAT <mod><Devname> ( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char       *taskDevName,

OUT ccsERROR         *error )

Two optional hooks might be implemented that are invoked when

starting resp. when terminating the task.

The function name syntax follows:

ccsCOMPL_STAT <mod><Devname>ProHook() for the prologue function, and

ccsCOMPL_STAT <mod><Devname>EpiHook() for the epilogue function,

same parameters as the core function.

The method lsfTaskDevTrigger() is provided that triggers

asynchronously the task, if running.

8 - Building the code

On both <mod>/ws/src and <mod>/lcu/src, invoke:

make clean all man install

9 - Building the environments

The instantiation of the database point associated to the Software

Device is realized from within the file:

$VLTDATA/ENVIRONMENTS/<env>/dbl/DATABASE.db

#define <mod>DB_ROOT "<< database absolute path >>"

#include "<mod>.db"

On the LCU environment, the file 'bootScript' shall be updated

that contains the modules: lcc, cai, scan, too, mcm, lsf and <mod>

The file 'devicesFile' shall be updated as well with the entry

corresponding to the new Software Device:

"<mod>" "<mod>Server" 1 0 0 1

NB: any Software Device to be controlled by another Software Device

MUST NOT be declared in this list.

10 - Database configuration file

The default database configuration file <mod>/ws/config/<mod>.dbcfg

shall be updated to store the complete final device configuration.

The backup of the database configuration is made by the utility

lsfConfig.

Example:

From the directory of <mod>/ws/config

> lsfBackup -e <lcuEnv> -m mod

Generating input file './mod.inp' ... done

Performing database backup into  './mod.dbcfg' ... done

1.24u 49.41s 1:59.60 42.3%

>

SEE ALSO

lsfCreate(1), lsfConfig(1), lsfBackup(1)

    - lsfCreate(1)

NAME

lsfCreate - Create a new module from lsftpl template

SYNOPSIS

lsfCreate [-f] <modName>

DESCRIPTION

This script creates a module directory <modName> from the

template module 'lsftpl'.

It checks that the new module is not already existing in the

CMM Archive.

It renames all files with the new prefix.

It replaces all occurences of 'lsftpl' by 'modName' in lower

and upper case.

Finally it creates the file <modName>/ChangeLog with one entry.

Options:

-f : the module is created anyway

the existing module 'modName' is renamed 'modName-old'

the module is created even if already in the CMM archive

ENVIRONMENT

CMM

RETURN VALUES

0 on SUCCESS

1 on Failure

CAUTIONS

The module is not created if it already exists under the CMM archive

or if the directory 'modName' already exists. Option -f bypasses these

checks.

EXAMPLES

> lsfCreate app

Copying template module 'lsftpl' to ./app ... done

Renaming all files with prefix 'app' ... done

Replacing pattern 'lsftpl' by 'app' ... done

1.24u 49.41s 1:59.60 42.3%

Module creation 'app' ... done

>

SEE ALSO

lsf(1), cmmCopy

    - lsfConfig(1)

NAME

lsfConfig - Configure the application

SYNOPSIS

lsfConfig <modName>

DESCRIPTION

This script configures the module <modName> according to the

description file <modName>/ws/config/<modName>.cfg

It generates the files:

- <modName>/ws/config/<modName>.dbcfg

- <modName>/lcu/include/<modName>DeviceList.h

- <modName>/lcu/dbl/<modName>.db

- <modName>/lcu/src/Makefile

It removes the unused files.

Finally it adds a log entry in <modName>/ChangeLog.

The syntax of the configuration file must follow:

LSF.MODULE.NAME     <modName>

LSF.DEVICE.NUM      <numDevices>

LSF.DEVICEi.NAME    <devName>

LSF.DEVICEi.TYPE    analog | digital | motor | serial | taskdev | softdev

LSF.DEVICEi.CLASS   <dbClass>

LSF.DEVICEi.PARAM   <parameter value>[, <parameter value>]

LSF.COMMAND.NUM     <numCommands>

LSF.COMMANDi.NAME   <cmdName>

LSF.COMMANDi.DEST   server | analog | digital | motor | serial | taskdev | softdev

LSF.COMMANDi.GROUP  PUBLIC | MAINTENANCE | TEST

LSF.COMMANDi.ENTRY  <entryFct>

LSF.COMMANDi.INVOKE FUNCTION | TASK

LSF.COMMANDi.OPTION <task parameters> (without REGISTER option)

FILES

<modName>/ws/config/<modName>.cfg

RETURN VALUES

0 on SUCCESS

1 on Failure

CAUTIONS

The script shall be run only once from the parent directory.

The deleted files can not be restored.

EXAMPLES

> lsfConfig app

Parse file 'app/ws/config/app.cfg' ... done

Found 16 devices:

2 Motor    : M1 M2

8 Signal   : AI1 AO3 AO4 AI8 DO1 DI2 DI3 DO4

3 Serial   : tyCo1 com1 com2

2 Softdev  : swd soft

1 Taskdev  : bgTask

Found 10 PUBLIC commands:

1 Motor    : MOVEA

4 Signal   : RASIG WASIG RDSIG WDSIG

2 Serial   : RSCOM WSCOM

2 Softdev  : SOFTCMD SWDCMD

1 Server   : MAINCMD

Found 0 MAINTENANCE command

Found 0 TEST command

Generate 'app/ws/config/app.dbcfg' ... done

Generate 'app/lcu/include/appDeviceList.h' ... done

Generate 'app/lcu/dbl/app.db' ... done

Generate 'app/lcu/src/Makefile' ... done

Generate 'app/lcu/src/appPublic.cdt' ... done

Generate 'app/lcu/src/appServer.cit' ... done

Generate 'app/lcu/src/appMotor.cit' ... done

Generate 'app/lcu/src/appSignal.cit' ... done

Generate 'app/lcu/src/appSerial.cit' ... done

Generate 'app/lcu/src/appSoftdev.cit' ... done

Move unused files to ./lcu/tmp ... done

Add change log entry ... done

>>> Remember to edit the CDT files ...

Module configuration for 'app' ... done"

>

SEE ALSO

lsf(1), lsfCreate(1)

BUGS

    - lsfBackup(1)

NAME

lsfBackup - Save LCU database configuration into file

SYNOPSIS

lsfBackup -e <lcuEnv> -m <modName> [-o <fileName>]

DESCRIPTION

This script saves the content of the LCU database to a file.

It generates a ".dbcfg" type file containing all the configuration

attributes of the module.

Arguments:

lcuEnv  : LCU environment name

modName : module name (alias of the database top point)

fileName: output file name (default to "./<modName>.dbcfg"

FILES

ENVIRONMENT

RETURN VALUES

0 on SUCCESS

1 on Failure

CAUTIONS

EXAMPLES

> lsfBackup -e lcuEnv -m app

Generating input file './app.inp' ... done

Performing database backup into  './app.dbcfg' ... done

1.24u 49.41s 1:59.60 42.3%

>

SEE ALSO

dbBackup

BUGS

  - Code

    - lsfServer(3)

NAME

lsfServer - Top level control of the application

SYNOPSIS

#include "lsfServer.h"

DESCRIPTION

This class provides the methods for the handling of the server process.

PUBLIC METHODS

ccsCOMPL_STAT lsfInitAll

( IN  const char      *swdName,

OUT lsfSERVER_DATA **pServerData )

This method is the constructor of the application server <swdName>.

It shall be invoked at boot time from the bootScript(5) as part

of the application file <swdName>.boot(3).

It receives the address of the pointer to the application global

data <pServerData>.

The method loads the database from the file <swdName>.dbcfg

located under VLTROOT/config or $INTROOT/config.

It invokes then the server constructor and finally spawns the

command interpreter with the associated CDT and CIT files.

vltLOGICAL    lsfServerGetInit

( IN lsfSERVER_DATA *serverData )

This method returns the initialization status of the device.

vltLOGICAL    lsfServerGetSim

( IN  lsfSERVER_DATA *serverData )

This method returns the simulation status of the device.

lccDEV_MODE   lsfServerGetState

( IN  lsfSERVER_DATA *serverData )

const char   *lsfServerGetStateName

( IN  lsfSERVER_DATA *serverData )

These methods return the state and state's name name of the device.

vltINT32      lsfServerGetSubState

( IN  lsfSERVER_DATA *serverData )

const char   *lsfServerGetSubStateName

( IN  lsfSERVER_DATA *serverData )

These methods return the subState and subState's name of the device.

ccsCOMPL_STAT lsfServerCheckOnline

( IN  lsfSERVER_DATA *serverData,

OUT vltLOGICAL     *online,

OUT ccsERROR       *error )

This method returns the flag <online> indicating if the device

is in state ONLINE when True.

lsfSERVER_DATA *lsfServerGetGlobalData

( IN  const char     *swdName,

OUT ccsERROR       *error )

This method returns the pointer to the global data structure of

the device.

ccsCOMPL_STAT lsfServerGetSwDeviceName

( OUT char           *swdName,

OUT ccsERROR       *error )

This method returns the name of the device from the task name of

the commmand interpreter process.

vltLOGICAL    lsfServerIsALccDevice

( IN  lsfSERVER_DATA *serverData )

This method returns the flag indicating if the device is registered

in the LCC Device Table.

Any Software Device controlled by another Software Device

must NOT be registered in LCC.

PROTECTED METHODS

void          lsfServerDestructor

( IN  lsfSERVER_DATA **pServerData )

This method is the object destructor. All resources are released.

void          lsfServerGetHooks

( IN  lsfSERVER_DATA  *serverData )

This method resolves the addresses of all existing hook functions

provided by the application. It is invoked once at Init time.

The function entry points are retrieved from the global

symbol table.

ccsCOMPL_STAT lsfServerCallHook

( IN  lsfSERVER_DATA  *serverData,

IN  lsfHOOKFUNC_IDX  cmd,

IN  lsfHOOKTYPE_IDX  typ,

OUT ccsERROR        *error )

This method resolves the hook function <typ> [Pro,Mid or Epi]

associated to the command indexed by <cmd>, if resolved.

ccsCOMPL_STAT lsfServerReadConfig

( IN  lsfSERVER_DATA *serverData,

OUT ccsERROR       *error )

This method reads the device configuration from the database.

ccsCOMPL_STAT lsfServerSetInit

( IN  lsfSERVER_DATA *serverData,

IN  vltLOGICAL      value,

OUT ccsERROR       *error )

These methods set/return the initialization status of the device.

ccsCOMPL_STAT lsfServerSetSim

( IN  lsfSERVER_DATA *serverData,

IN  vltLOGICAL      value,

OUT ccsERROR       *error )

This method sets the simulation status of the device.

ccsCOMPL_STAT lsfServerTakeInitSem

( IN  lsfSERVER_DATA *serverData )

void          lsfServerGiveInitSem

( IN  lsfSERVER_DATA *serverData )

These methods control the initialization semaphore associated

to the device. It prevents multiple invocation.

ccsCOMPL_STAT lsfServerTakeCmdSem

( IN  lsfSERVER_DATA *serverData )

void          lsfServerGiveCmdSem

( IN  lsfSERVER_DATA *serverData )

These methods control the command semaphore associated

to the device. It prevents multiple invocation.

void          lsfServerWakeupMonitor

( IN lsfSERVER_DATA *serverData  )

This method triggers the monitor task for state update

vltINT32      lsfServerGetCmdTimeout

( IN  lsfSERVER_DATA *serverData )

void          lsfServerSetCmdTimeout

( IN  lsfSERVER_DATA *serverData,

IN  vltINT32        timeout )

These methods set/get the timeout value associated to the commands.

ccsCOMPL_STAT lsfServerSwitchState

( IN  lsfSERVER_DATA *serverData,

IN  lccDEV_MODE     newDevState,

OUT ccsERROR       *error )

This method instructs LCC of the new state <newDevState> of the

device.

int           lsfStartupTask

( IN  char           *name,

IN  vltINT32        priority,

IN  vltINT32        stackSize,

IN  FUNCPTR         entryPoint,

IN  void           *param,

OUT ccsERROR       *error )

This method spawns the task <name> with the appropiate parameters,

and checks the task has synchronized with the caller. The task entry

function must comply with the prototype lsfTASK_FUNCTION.

typedef void lsfTASK_FUNCTION ( IN void          *parameter,

IN SEM_ID        *syncSem,

IN ccsCOMPL_STAT *status )

The first argument is application dependent. The semaphore <syncSem>

must be given by the task within 1 second, and the task status

<status> shall be set accordingly to SUCCESS or FAILURE.

PRIVATE METHODS

static ccsCOMPL_STAT lsfServerUpdateState

( IN  lsfSERVER_DATA  *serverData,

OUT ccsERROR        *error )

static ccsCOMPL_STAT lsfServerUpdateSubState

( IN  lsfSERVER_DATA  *serverData,

OUT ccsERROR        *error )

These 2 methods are responsible for updating the state and subState

of the device. They query the individual states and subStates of all

registered devices.

The device state is set to the lower state of the devices.

The subState is set to :

- lsfSUBSTATE_ERROR if any device is in that subState.

- lsfSUBSTATE_INITIALIZING if any device is in that subState.

- lsfSUBSTATE_IDLE  if all devices are in that subState

- lsfSUBSTATE_ACTIVE if any device is not lsfSUBSTATE_IDLE and

the device subState is not lsfSUBSTATE_IDLE

- lsfSUBSTATE_<ACTION> if a device is in that subState and

the device subState is lsfSUBSTATE_IDLE

The following subStates are dedicated:

- lsfSUBSTATE_MOVING for a motor when moving

- lsfSUBSTATE_MONITORING for a device monitoring a piece of hardware

In addition, the subState lsfSUBSTATE_TIMEOUT indicates a timeout

occured.

Both lsfSUBSTATE_TIMEOUT and lsfSUBSTATE_ERROR are transient

subStates that reset to lsfSUBSTATE_IDLE with the next action.

static ccsCOMPL_STAT lsfServerInitDb

( IN  const char      *swdName,

OUT ccsERROR        *error )

This method is invoked at boot time by lsfInitAll(). It restores

the content of the device database branch from the file

<swdName>.dbcfg located under $VLTROOT/config or $INTROOT/config.

static void lsfServerMonitor

( IN  void            *pData,

IN  SEM_ID          *pSyncSem,

IN  ccsCOMPL_STAT   *pStatus )

This method is invoked as a task (see lsfStartupTask) at construction

time. It is responsible for updating the device global state and

subState at the given rate (see lsfSERVER(5)).

Note: be aware that a high polling rate is CPU consuming and might

impact on the overall behaviour of the system.

static ccsCOMPL_STAT lsfServerConstructor

( IN  const char      *swdName,

OUT lsfSERVER_DATA **pServerData,

OUT ccsERROR        *error )

This method is the constructor of the software device. It invokes

the control contructor (see lsfControl(3)) and spawns the monitor.

PRIVATE DATA MEMBERS

typedef struct

{

void        *ctrlData;     Ptr to Control Data structure

lccDEV_MODE  state;        Module state

vltINT32     subState;     Module sub-state

vltLOGICAL   init;         True if initialized

vltLOGICAL   simulation;   True if in simulation

vltINT32     numDevices;   number of controlled devices (registered)

SEM_ID       cmdSem;       semaphore for sending commands

vltINT32     cmdTimeout;   timeout for sending commands

SEM_ID       initSem;      semaphore for init command

vltLOGICAL   lccDevice;    SW device is a LCC device

int          monTid;       Monitoring Task Id

SEM_ID       monSem;       semaphore for monitor task period

int          monPeriod;    Monitoring Task Period (ticks)

ccsERROR     error;        error stack for cmdInit()

char        *attrTable;    cai DB attribute table

vltBYTES20   swdName;      Name of the SW device

void        *fctHook[8][3]; Hook function addresses

} lsfSERVER_DATA;

FILES

$INTROOT/config/<modName>.dbcfg

DATABASE

The class lsfDB_SERVER is a sub-class of lsfDB_SOFTDEV.

It is partially mapped by the data structure lsfSERVER_DATA.

SEE ALSO

lsfControl(3), lsfDevice(3), lsfDB_DEVICE(5)

    - lsfStandard(3)

NAME

lsfStandard - Standard commands

SYNOPSIS

#include "lsfStandard.h"

DESCRIPTION

This class provides the methods implementing the high level default

behaviour of the standard commands.

For each of the standard commands, up to 3 hook functions might be

defined following the syntax:

<mod>Std<cmd><typ>Hook(lsfSERVER_DATA *serverData, ccsERROR *error)

where

<mod> is the module name,

<cmd> is the standard command name (Init, Online etc...)

<typ> is the hook index (Pro,Mid,Epi)

Pro: hook is invoked when entering the method,

Mid: hook is invoked between Software and Hardware device control

Epi: hook is invoked before leaving the method.

The first (Pro) and last (Epi) are called regardless on the number

of devices. The middle hook (Mid) is only invoked from within the control

object.

PUBLIC METHODS

ccsCOMPL_STAT lsfStdInit

ccsCOMPL_STAT lsfStdStandby

ccsCOMPL_STAT lsfStdOnline

ccsCOMPL_STAT lsfStdStop

ccsCOMPL_STAT lsfStdExit

ccsCOMPL_STAT lsfStdStartSim

ccsCOMPL_STAT lsfStdStopSim

ccsCOMPL_STAT lsfStdSelfTest

ccsCOMPL_STAT lsfStdTest

( IN  lsfSERVER_DATA *serverData,

IN  const char     *devName,

OUT ccsERROR       *error )

These methods implement the high level behaviour associated

to the standard commands. They invoke the associated control

method for the specified device <devName>.

If <devName> is set to the software device name, the method

affects all devices registered in that software device, otherwise

the action is restricted to the one device.

At completion, the methods update the global state accordingly.

ccsCOMPL_STAT lsfStdGetInit

( IN  lsfSERVER_DATA *serverData,

OUT vltLOGICAL     *init,

OUT ccsERROR       *error )

This method returns the initialization status.

ccsCOMPL_STAT lsfStdGetSimulation

( IN  lsfSERVER_DATA *serverData,

OUT vltLOGICAL     *simulation,

OUT ccsERROR       *error )

This method returns the value of the simulation flag.

ccsCOMPL_STAT lsfStdGetVersion

( OUT char           *version,

OUT ccsERROR       *error )

This method returns the software version of the module LSF.

PRIVATE METHODS

static const char *lsfCheckDevice

( IN  lsfSERVER_DATA *serverData,

IN  const char     *devName,

OUT ccsERROR       *error )

This method checks if the given device is known:

- name of the software device or "all", it returns "all"

- name of a controlled device, it returns the name if defined.

- unknown name, it returns NULL.

PRIVATE DATA MEMBERS

These methods use the data structure <serverData>.

COMMANDS

The public methods are the API functions associated to the

ACI for the Standard Commands.

RETURN VALUES

SUCCESS if everything ok

FAILURE, the error structure is updated with:

lsfERR_DEVICE           : unknown device name or

module name mismatch

lsfERR_MULTIPLE_COMMAND : multiple invocation not allowed

SEE ALSO

lsfServer(3)

    - lsfSignalHandlers(3)

NAME

lsfHandleBreak, lsfHandleKill - signal handler for BREAK and KILL signals

SYNOPSIS

#include "lsfSignalHandlers.h"

void lsfHandleKill(void);

void lsfHandleBreak(void);

DESCRIPTION

lsfHandleBreak() and lsfHandleKill() are the 2 signal handler methods

installed at boot time by lsfInitAll(). These methods retrieve the

name of the affected software device from the server task name before

performing the corresponding actions (STOP, resp. EXIT).

RETURN VALUES

None

SEE ALSO

lsfStandard(3), lsfServer(3)

    - lsfControl(3)

NAME

lsfControl - Device control

SYNOPSIS

#include "lsfControl.h"

DESCRIPTION

This class provides the methods dedicated to the control

of the devices. It is driven by the deviceTable. This table is read

at init time.

For each of the standard commands, the associated control method

invokes the device methods for each type of device assuming devices of

that type are registered and not ignored.

First the sub-software devices are invoked, then the hardware

devices whereby the digital and analogue signals are processed first,

followed by the motors.

For each of the standard commands, up to 3 hook functions might be

defined following the syntax:

<mod>Std<cmd><typ>Hook(lsfCONTROL_DATA *controlData, ccsERROR *error)

where

<mod> is the module name,

<cmd> is the standard command name (Init, Online etc...)

<typ> is the hook index (Pro,Mid,Epi)

Pro: hook is invoked when entering the method,

Mid: hook is invoked between Software and Hardware device control

Epi: hook is invoked before leaving the method.

The first (Pro) and last (Epi) are called from within the standard

object regardless on the number of devices. The middle hook (Mid) is

only invoked from within this object.

PROTECTED METHODS

ccsCOMPL_STAT lsfControlConstructor

( IN  const char      *swdName,

IN  lsfDEVICE_DATA  *serverData,

OUT ccsERROR        *error )

This method is the object constructor. It invokes the constructors

of all devices registered in the device table.

void lsfControlDestructor

( IN lsfDEVICE_DATA *serverData )

This method is the object destructor. It invokes the destructors

of all registered devices. All resources are released.

const char *lsfControlGetSwdName

( IN  void *pData )

This method returns the name of the software device (= module name)

void *lsfControlGetDevData

( IN lsfCONTROL_DATA *controlData,

IN int devType )

This method returns a pointer to the device data structure

of the given device type.

int lsfControlGetDevType

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName )

This method returns the type of the given device.

int lsfControlGetDevNamesOfType

( IN  lsfCONTROL_DATA *controlData,

IN  int              devType,

IN  const char      *devNames[] )

This method returns the number of devices of the given type.

The array <devNames[]> contains the names of all the matching

devices.

ccsCOMPL_STAT lsfControlInit

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error)

ccsCOMPL_STAT lsfControlStandby

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error )

ccsCOMPL_STAT lsfControlOnline

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error )

ccsCOMPL_STAT lsfControlStop

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error)

ccsCOMPL_STAT lsfControlOff

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error)

ccsCOMPL_STAT lsfControlExit

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error )

ccsCOMPL_STAT lsfControlSimulat

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error )

ccsCOMPL_STAT lsfControlStopsim

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error )

ccsCOMPL_STAT lsfControlSelftest

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error )

ccsCOMPL_STAT lsfControlTest

( IN  lsfCONTROL_DATA *controlData,

IN  const char      *devName,

OUT ccsERROR        *error )

These methods perform the actions corresponding to the standard

behaviour.

PRIVATE METHODS

static int lsfDecodeDeviceType

(char *type)

This method returns the associated device type associated to

the device type name.

static ccsCOMPL_STAT lsfControlReadDevTbl

( IN  lsfCONTROL_DATA *controlData,

OUT ccsERROR        *error )

This method reads the database attribute control:deviceTable

and sets the private data member <deviceTable>.

PRIVATE DATA MEMBERS

The data structure lsfCONTROL_ENTRY maps the database attribute

:control.deviceTable.

typedef struct

{

vltBYTES20 name;                 Name of the device

vltINT32   type;                 Type of the device

vltINT32   startPhase;           Sequence order for increasing/

vltINT32   stopPhase;              decreasing the state

vltLOGICAL simulation;           True when device is in Simulation

vltLOGICAL ignored;              True when device is ignored

} lsfCONTROL_ENTRY;

The data structure lsfCONTROL_DATA holds all the necessary information

for the control of the devices.

typedef struct

{

void             *ctrlData;      Ptr to self

void             *srvrData;      Ptr to parent structure

int               numDevices[lsfDEVICE_NUM_TYPES+1];

Number of devices of each type

lsfCONTROL_ENTRY *deviceTable;

Device table (mirror of DB)

lsfDEVICE_DATA   *deviceData[lsfDEVICE_NUM_TYPES+1];

Device data

char             *attrTable;     LCC cai internal

void             *fctHook[8][3]; Hook function addresses

} lsfCONTROL_DATA;

DATABASE

The class lsfCONTROL is the class holding the description of the

device. It consists of the deviceTable attribute that stores the

description of each device to be controlled:

vltBYTES20 name;               Name of the device

vltBYTES20 type;               Type of the device

vltINT32   startPhase;         Sequence order for increasing/

vltINT32   stopPhase;            decreasing the state

vltLOGICAL simulation;         Device is in Simulation

vltLOGICAL ignored;            Device is ignored

For each kind of device a point is instanciated from the class lsfDEVICE

that holds the information of that kind of devices (state, subState

etc...). These points are instanciated dynamically at build time

depending on the number of devices of a kind.

RETURN VALUES

The control sequence is broken on the first failure.

The method returns FAILURE and an error is logged.

CAUTIONS

Start/Stop Phase handling mechanism is not yet implemented.

SEE ALSO

lsfCONTROL(5)

    - lsfDevice(3)

NAME

lsfDevice - Common device handling methods

SYNOPSIS

#include "lsfDevice.h"

DESCRIPTION

This class provides methods dedicated to the handling of devices.

The allows other objects to query device private information and

to set the device state/subState.

PUBLIC METHODS

vltLOGICAL    lsfDeviceIsDefined   ( IN  void       *pData,

IN  const char *devName );

vltLOGICAL    lsfDeviceIsIgnored   ( IN  void       *pData,

IN  const char *devName );

vltLOGICAL    lsfDeviceIsSimulated ( IN  void       *pData,

IN  const char *devName );

vltLOGICAL    lsfDeviceIsInState   ( IN  void       *pData,

IN  vltINT32    state );

These 4 methods query the device status.

PROTECTED METHODS

ccsCOMPL_STAT lsfDeviceSetStatus   ( IN  void       *pData,

IN  vltINT32    devIdx,

IN  vltINT32    state,

IN  vltINT32    subState,

IN  vltINT32    init,

OUT ccsERROR   *error );

This method sets the device status to the given values for:

the <state>, the <subState> and the <init> flag.

The index <devIdx> refers to the selected device when > 0

or to the device group when zero.

ccsCOMPL_STAT lsfDeviceSetState    ( IN  void       *pData,

IN  vltINT32    devIdx,

IN  vltINT32    state,

OUT ccsERROR   *error );

Idem dito for the <state> only.

ccsCOMPL_STAT lsfDeviceSetSubState ( IN  void       *pData,

IN  vltINT32    devIdx,

IN  vltINT32    subState,

OUT ccsERROR   *error );

Idem dito for the <subState> only.

ccsCOMPL_STAT lsfDeviceSetInit     ( IN  void       *pData,

IN  vltINT32    devIdx,

IN  vltLOGICAL  init,

OUT ccsERROR   *error );

Idem dito for the <init> flag only.

PRIVATE DATA MEMBERS

typedef struct

{

vltINT32         state;            State of the device

vltINT32         subState;         Associated sub-state

vltLOGICAL       init;             True when Initialized

vltLOGICAL       simulation;       True when in Simulation

vltINT32         timeout;          Device action timeout in seconds

} lsfDEVICE_ENTRY;

typedef struct

{

void            *ctrlData;         Ptr to the CONTROL_DATA structure

void            *userData;         Ptr to User data

vltINT32         state;            State, SubState and Init/Sim flag of

vltINT32         subState;           all the devices of this type

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;          Timeout in seconds

vltINT32         numDevices;       Number of devices of this type

vltBYTES20      *deviceNames;      Names of the devices

char            *attrTable[lsfMAX_DEVICES+1]; LCC cai internal

lsfDEVICE_ENTRY *deviceTable[lsfMAX_DEVICES]; Device individual data

} lsfDEVICE_DATA;

DATABASE

The class lsfDB_DEVICE is the generic class for device description:

It is partially mapped by the data structure lsfDEVICE_ENTRY.

Each type of device is represented in the database branch of the

Software Device by a point instanciated from lsfDB_DEVICE

and located under :control.

Each device of a kind is represented in the database by a point

instanciated from a sub-class of lsfDB_DEVICE and

located under :control:<devType>.

SEE ALSO

lsfControl(3), lsfDB_DEVICE(5)

    - lsfSignal(3)

NAME

lsfSignal - Control digital and analogue signals

SYNOPSIS

#include "lsfSignal.h"

DESCRIPTION

This class provides the methods for the control of digital and

analogue signals. It implements the default behaviour for all

the standard commands.

A user-defined hook function is foreseen to be invoked at

completion of the signal initialization.

This hook shall follow the syntax:

ccsCOMPL_STAT <mod>SignalInitHook(lsfSIGNAL_DATA *signalData,

const char     *signalName,

ccsERROR       *error)

In addition, it offers the 4 methods for reading and setting

digital and analogue signals as a whole or individually.

PUBLIC METHODS

ccsCOMPL_STAT lsfSignalReadAnalog

( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

OUT char           *names[],

OUT vltFLOAT       *value,

OUT ccsERROR       *error )

This method reads the specified analog signal value(s) and

stores it/them in the array pointed to by <value>. The names

of the signals are listed and returned to the caller via the

array <names[]>.

If <signalName> is NULL or "all", all input analogue signals

will be read.

ccsCOMPL_STAT lsfSignalWriteAnalog

( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

IN  vltFLOAT       *value,

OUT ccsERROR       *error )

This method sets the specified analog signal <signalName>

to the value pointed to by <value>.

If <signalName> is NULL or "all", all output analogue signals

will be set to the values pointed to by <value> in the order

they are registered.

ccsCOMPL_STAT lsfSignalReadDigital

( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

OUT char           *names[],

OUT vltUINT32      *value,

OUT ccsERROR       *error )

This method reads the specified digital signal value(s) and

stores it/them in the array pointed to by <value>. The names

of the signals are listed and returned to the caller via the

array <names[]>.

If <signalName> is NULL or "all", all input digital signals

will be read.

ccsCOMPL_STAT lsfSignalWriteDigital

( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

IN  vltUINT32      *value,

OUT ccsERROR       *error )

This method sets the specified digital signal <signalName>

to the value pointed to by <value>.

If <signalName> is NULL or "all", all output digital signals

will be set to the values pointed to by <value> in the order

they are registered.

PROTECTED METHODS

ccsCOMPL_STAT lsfSignalConstructor

( IN  const char      *swdName,

IN  lsfCONTROL_DATA *controlData,

IN  const char      *signalNames[],

OUT ccsERROR        *error )

This is the constructor of the signal object. It affects all the

signals given in the list <signalNames[]> (NULL terminated).

void          lsfSignalDestructor

( IN  lsfCONTROL_DATA *controlData )

This method is the object destructor. All resources are released.

ccsCOMPL_STAT lsfSignalInit

ccsCOMPL_STAT lsfSignalStandby

ccsCOMPL_STAT lsfSignalOnline

ccsCOMPL_STAT lsfSignalStop

ccsCOMPL_STAT lsfSignalOff

ccsCOMPL_STAT lsfSignalExit

ccsCOMPL_STAT lsfSignalSimulat

ccsCOMPL_STAT lsfSignalStopsim

ccsCOMPL_STAT lsfSignalSelftest

ccsCOMPL_STAT lsfSignalTest

( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

OUT ccsERROR       *error )

These methods implement the behaviour of the standard commands.

If <signalName> is NULL, or "all" all the signals will be

affected, otherwise only the specified one.

PRIVATE METHODS

static ccsCOMPL_STAT lsfSignalCallHook

( IN  lsfSIGNAL_DATA *signalData,

IN  const char     *signalName,

IN  const char     *signalFct,

OUT ccsERROR       *error )

This method invokes the user-defined hook function

<mod>Signal<signalFct>Hook(<signalData>,<signalName>,error).

It is invoked at Init completion only.

PRIVATE DATA MEMBERS

The data stucture lsfSIGNAL_ENTRY is a sub-class of lsfDEVICE_ENTRY.

typedef struct

{

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

Additional data members

ioTYPE           ioType;        Type of signal (Digital/Analog)

ioIN_OUT         direction;     Signal direction (Input/Output)

dbSYMADDRESS     dbAddr;        DB path to Signal point

ioDIRADDRESS     ioAddr;        IO Direct address

} lsfSIGNAL_ENTRY;

The data stucture lsfSIGNAL_DATA is a sub-class of lsfDEVICE_DATA.

typedef struct

{

void            *ctrlData;

void            *userData;

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

vltINT32         numSignals;                    Number of signals

vltBYTES20       *signalNames;                  Signal names

(dynamic allocated)

char             *attrTable[lsfMAX_DEVICES+1];

lsfSIGNAL_ENTRY  *signalTable[lsfMAX_DEVICES];  Signal data

} lsfSIGNAL_DATA;

DATABASE

The class lsfDB_SIGNAL is a sub-class of lsfDB_DEVICE.

It is partially mapped by the data structure lsfSIGNAL_ENTRY.

Each signal is represented in the database by a point under the

main signal point :control:signal, which is partially mapped

by the data structure lsfSIGNAL_DATA.

The 2 sub-classes lsfDB_ANALOG and lsfDB_DIGITAL are specialized classes

for Analogue, resp. Digital signals. They are instanciating the LCC

classes lccANALOG_SIGNAL and lccDIGITAL_SIGNAL resp.

SEE ALSO

lsfDB_SIGNAL(5)

    - lsfMotor(3)

NAME

lsfMotor - Control motors

SYNOPSIS

#include "lsfMotor.h"

DESCRIPTION

This class provides the methods for the control of motors.

It implements the default behaviour for all the standard commands.

In addition, it offers 2 methods for performing absolute motions

on one or all motors in parallel.

PUBLIC METHODS

vltINT32 lsfMotorGetState    ( IN  lsfMOTOR_DATA *motorData,

IN  const char    *motorName )

vltINT32 lsfMotorGetSubState ( IN  lsfMOTOR_DATA *motorData,

IN  const char    *motorName )

These 2 methods return the state, resp. sub-state of the motors

specified by <motorName>.

ccsCOMPL_STAT lsfMotorMoveAbs

( IN  lsfMOTOR_DATA *motorData,

IN  const char    *motorName,

IN  vltINT32       target[],

IN  const char    *unit[],

IN  vltLOGICAL     wait,

OUT ccsERROR      *error )

This methods initiates an absolute motion to the position

<target> for all the motors specified by <motorName>. The

position unit is specified by <unit> in the corresponding order.

The flag <wait> indicates whether the function shall wait for

the motion completion, or not.

ccsCOMPL_STAT lsfMotorWaitMove

( IN  lsfMOTOR_DATA *motorData,

IN  const char    *motorName,

OUT char          *names[],

OUT vltDOUBLE     *position,

OUT char          *units[],

OUT ccsERROR      *error )

This method is based on the API function motWaitMove() that

waits until all motors specified by <motorName> have completed

their motion. It returns on the first error.

PROTECTED METHODS

ccsCOMPL_STAT lsfMotorConstructor

( IN  const char      *swdName,

IN  lsfCONTROL_DATA *controlData,

IN  const char      *motorNames[],

OUT ccsERROR        *error )

This is the constructor of the motor object. It affects all the

motors given in the list <motorNames[]> (NULL terminated).

void          lsfMotorDestructor

( IN  lsfCONTROL_DATA *controlData )

This method is the object destructor. It deinstalls all

of all registered motors. All resources are released.

ccsCOMPL_STAT lsfMotorInit

ccsCOMPL_STAT lsfMotorStandby

ccsCOMPL_STAT lsfMotorOnline

ccsCOMPL_STAT lsfMotorStop

ccsCOMPL_STAT lsfMotorOff

ccsCOMPL_STAT lsfMotorExit

ccsCOMPL_STAT lsfMotorSimulat

ccsCOMPL_STAT lsfMotorStopsim

ccsCOMPL_STAT lsfMotorSelftest

ccsCOMPL_STAT lsfMotorTest   ( IN  lsfMOTOR_DATA  *motorData,

IN  const char     *motorName,

OUT ccsERROR       *error )

These methods implement the behaviour of the standard commands.

If <motorName> is NULL, or "all" all the motors will be

affected, otherwise only the specified one.

ccsCOMPL_STAT lsfMotorWaitInit

( IN  lsfMOTOR_DATA *motorData,

IN  const char    *motorName,

OUT ccsERROR      *error )

This method is based on the API function motWaitInit() that

waits until all motors specified by <motorName> have completed

their initialization sequence. It returns on the first error.

ccsCOMPL_STAT lsfMotorSetSubState

( IN  lsfMOTOR_DATA *motorData,

OUT ccsERROR      *error )

This method sets the sub-states of all registered motors as a

function of their resp. initStatus and motionStatus.

PRIVATE DATA MEMBERS

The data stucture lsfMOTOR_ENTRY is a sub-class of lsfDEVICE_ENTRY.

typedef struct

{

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

Additional data members

motSTATUS       *motorStatus;   Ptr to motorStatus array

from lsfMOTOR_DATA

} lsfMOTOR_ENTRY;

The data stucture lsfMOTOR_DATA is a sub-class of lsfDEVICE_DATA.

typedef struct

{

void            *ctrlData;

void            *userData;

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

vltINT32         numMotors;                    Number of motors

vltBYTES20      *motorNames;                   Motor names

(dynamic allocated)

char            *attrTable[lsfMAX_DEVICES+1];

lsfMOTOR_ENTRY  *motorTable[lsfMAX_DEVICES];   Motor data

Additional data members

motHANDLE        motorHandle[lsfMAX_DEVICES+1]; List of motor

handles [0 terminated]

motSTATUS       *motorStatus;                  Array of motor status

(dynamic allocated)

} lsfMOTOR_DATA;

DATABASE

The class lsfDB_MOTOR is a sub-class of lsfDB_DEVICE.

It is partially mapped by the data structure lsfMOTOR_ENTRY.

Each motor is represented in the database by a point under the

main  warning: imsignal point :control:motor, which is partially mapped

by the data structure lsfMOTOR_DATA.

SEE ALSO

lsfDB_MOTOR(5)

    - lsfSerial(3)

NAME

lsfSerial - Control serial communication links

SYNOPSIS

#include "lsfSerial.h"

DESCRIPTION

This class provides the methods for the control of serial

communication links. It implements the default behaviour for all

the standard commands.

PUBLIC METHODS

ccsCOMPL_STAT lsfSerialRead  ( IN  lsfSERIAL_DATA *serialData,

IN  const char     *serialName,

OUT char           *names[],

OUT char           *buffer[],

OUT ccsERROR       *error)

This method reads data from the given serial communication

link <serialName> into the buffer pointed to by <buffer>.

If <serialName> is NULL or "all", all communication links

will be read in the order they are registered. The names of

devices read are returned in the array <names[]>. The data

buffers are ordered identically.

ccsCOMPL_STAT lsfSerialWrite ( IN  lsfSERIAL_DATA *serialData,

IN  const char     *serialName,

IN  char           *buffer[],

OUT ccsERROR       *error )

This method writes data pointed to by <buffer> to the given

serial communication link <serialName>.

If <serialName> is NULL or "all", all communication links

will be written in the order they are registered.

PROTECTED METHODS

ccsCOMPL_STAT lsfSerialConstructor

( IN  const char      *swdName,

IN  lsfCONTROL_DATA *controlData,

IN  const char      *serialNames[],

OUT ccsERROR        *error )

This is the constructor of the serial object. It affects all the

serials given in the list <serialNames[]> (NULL terminated).

void          lsfSerialDestructor

( IN  lsfCONTROL_DATA *controlData )

This method is the object destructor. All resources are released.

ccsCOMPL_STAT lsfSerialInit

ccsCOMPL_STAT lsfSerialStandby

ccsCOMPL_STAT lsfSerialOnline

ccsCOMPL_STAT lsfSerialStop

ccsCOMPL_STAT lsfSerialOff

ccsCOMPL_STAT lsfSerialExit

ccsCOMPL_STAT lsfSerialSimulat

ccsCOMPL_STAT lsfSerialStopsim

ccsCOMPL_STAT lsfSerialSelftest

ccsCOMPL_STAT lsfSerialTest

( IN  lsfSERIAL_DATA *serialData,

IN  const char     *serialName,

OUT ccsERROR       *error )

These methods implement the behaviour of the standard commands.

If <serialName> is NULL, or "all" all the serials will be

affected, otherwise only the specified one.

PRIVATE METHODS

static ccsCOMPL_STAT lsfSerialCallHook

( IN  lsfSERIAL_DATA *serialData,

IN  const char     *serialName,

IN  const char     *serialFct,

OUT ccsERROR       *error )

This method invokes the user-defined hook function

<mod>Serial<serialFct>Hook(<serialData>,<serialName>,error).

It is invoked at Init completion only.

PRIVATE DATA MEMBERS

The data stucture lsfSERIAL_ENTRY is a sub-class of lsfDEVICE_ENTRY.

typedef struct

{

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

Additional data members

vltINT32         fd;            File descriptor

vltLOGICAL       isTyCo;        True when TyCo device

} lsfSERIAL_ENTRY;

The data stucture lsfSERIAL_DATA is a sub-class of lsfDEVICE_DATA.

typedef struct

{

void            *ctrlData;

void            *userData;

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

vltINT32         numSerials;                    Number of serials

vltBYTES20       *serialNames;                  Serial names

(dynamic allocated)

char             *attrTable[lsfMAX_DEVICES+1];

lsfSERIAL_ENTRY  *serialTable[lsfMAX_DEVICES];  Serial data

} lsfSERIAL_DATA;

DATABASE

The class lsfDB_SERIAL is a sub-class of lsfDB_DEVICE.

It is partially mapped by the data structure lsfSERIAL_ENTRY.

Each serial communication link is represented in the database by a

point under the main serial point :control:serial, which is partially

mapped by the data structure lsfSERIAL_DATA.

The sub-classes lsfDB_RS232, lsfDB_RS422 and lsfDB_RS485 derive from lsfDB_SERIAL

whereby the protocol and handshake are overloaded.

SEE ALSO

lsfDB_SERIAL(5)

    - lsfTaskDev(3)

NAME

lsfTaskDev - Control Task Devices

SYNOPSIS

#include "lsfTaskDev.h"

DESCRIPTION

This class provides the methods for the control of task devices.

It implements the default behaviour for all the standard commands.

Function of the state, the task devices are activated/stopped.

PUBLIC METHODS

ccsCOMPL_STAT lsfTaskDevTrigger

( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

OUT ccsERROR        *error )

This method triggers the given task asynchronously.

If the timer is based on TIM, the activation occurs asynchronously

within the period; otherwise, the periodic delay is restarted

after activation.

STATUS        lsfTaskDevVerify

( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

OUT ccsERROR        *error )

This method returns the status of the given task device.

OK if running; ERROR if not. see taskIdVerify()

If <taskDevName> is "all" the return value is OK if all tasks

are running otherwise ERROR;

PROTECTED METHODS

ccsCOMPL_STAT lsfTaskDevConstructor

( IN  const char      *swdName,

IN  lsfCONTROL_DATA *controlData,

IN  const char      *taskDevNames[],

OUT ccsERROR        *error )

This is the constructor of the taskDev object. It affects all the

task devices given in the list <taskDevNames[]> (NULL terminated).

void          lsfTaskDevDestructor

( IN  lsfCONTROL_DATA *controlData )

This method is the object destructor. All resources are released.

ccsCOMPL_STAT lsfTaskDevInit

ccsCOMPL_STAT lsfTaskDevStandby

ccsCOMPL_STAT lsfTaskDevOnline

ccsCOMPL_STAT lsfTaskDevStop

ccsCOMPL_STAT lsfTaskDevOff

ccsCOMPL_STAT lsfTaskDevExit

ccsCOMPL_STAT lsfTaskDevSimulat

ccsCOMPL_STAT lsfTaskDevStopsim

ccsCOMPL_STAT lsfTaskDevSelftest

ccsCOMPL_STAT lsfTaskDevTest ( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

OUT ccsERROR        *error )

These methods forward the standard commands to the task

devices given in the list <taskDevName>.

If <taskDevName> is NULL, or "all" all the task devices will

be affected, otherwise only the specified one.

It updates the state and subState of the group.

PRIVATE METHODS

static ccsCOMPL_STAT lsfTaskDevLoop

( IN  lsfTASKDEV_DATA *taskDevData,

IN  int              tskIdx,

OUT ccsCOMPL_STAT   *tskStat,

IN  SEM_ID          *tskSync,

OUT ccsERROR        *error )

This method is the core of the task. Therefore the structure is

imposed. It invoked an optional prologue user function

<mod><taskDevName>ProHook(), then enters the infinite loop

in which it invokes the user core function <mod><taskDevName>()

When the task is told to terminate, it invoked the optional

epilogue user function <mod><taskDevName>EpiHook().

All the task hook functions must comply the prototype:

typedef ccsCOMPL_STAT lsfTASKDEV_HOOK

( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

OUT ccsERROR        *error)

The function names must follow the syntax:

<moduleName><taskDevName>["ProHook" | "" | "EpiHook"]()

The core function <moduleName><taskDevName>() is mandatory.

static ccsCOMPL_STAT lsfTaskDevActivate

( IN  lsfTASKDEV_DATA *taskDevData,

IN  int              tskIdx,

OUT ccsERROR        *error )

This method activates the task indexed <tskIdx>.

static ccsCOMPL_STAT lsfTaskDevSchedule

( IN  lsfTASKDEV_DATA *taskDevData,

IN  const char      *taskDevName,

IN  vltINT32         state,

OUT ccsERROR        *error )

This method schedules the tasks depending on their start state.

If the <state> is higher or equal the start state, the task is

activated, otherwise it is terminated.

PRIVATE DATA MEMBERS

The data stucture lsfTASKDEV_ENTRY is a sub-class of lsfDEVICE_ENTRY.

typedef struct

{

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

Additional data members

vltINT32      startState;       Miniumum state for schedule

vltLOGICAL    ignoreStop;       True when stop is ignored

vltLOGICAL    first;            True when starting

vltINT32      tcb,tid;          Task TCB for lccTaskLib

void         *fctHook[3];       Addresses of function hooks

} lsfTASKDEV_ENTRY;

The data stucture lsfTASKDEV_DATA is a sub-class of lsfDEVICE_DATA.

typedef struct

{

void             *ctrlData;

void             *userData;

vltINT32          state;

vltINT32          subState;

vltLOGICAL        init;

vltLOGICAL        simulation;

vltINT32          timeout;

vltINT32          numTaskDevs;         Number of tasks

vltBYTES20       *taskDevNames;        Names of the tasks

(dynamic allocated)

char             *attrTable[lsfMAX_DEVICES+1];

lsfTASKDEV_ENTRY *taskDevTable[lsfMAX_DEVICES];   TaskDev data

} lsfTASKDEV_DATA;

DATABASE

The class lsfDB_TASKDEV is a sub-class of lsfDB_DEVICE.

It is partially mapped by the data structure lsfTASKDEV_ENTRY.

Each task is represented in the database by a point under the

main point :control:taskDev, which is partially mapped by the data

structure lsfTASKDEV_DATA.

SEE ALSO

lsfDB_DEVICE(5)

    - lsfSoftDev(3)

NAME

lsfSoftDev - Control underlying Software Devices

SYNOPSIS

#include "lsfSoftDev.h"

DESCRIPTION

This class provides the methods for the control of software devices.

It implements the default behaviour for all the standard commands.

The standard commands are forwarded to the underlying software devices.

PUBLIC METHODS

ccsCOMPL_STAT lsfForwardCommand

( IN  lsfSOFTDEV_DATA *softDevData,

IN  const char      *softDevName,

IN  const char      *command,

IN  const char      *param,

OUT char            *reply,

OUT ccsERROR        *error )

This method forward a command to the software devices specified

by <softDevName>. If <softDevName> is NULL, or "all" all the

software devices will be affected, otherwise only the specified one.

The method returns the completion status of the command, thus waits

for all specified software devices to reply.

PROTECTED METHODS

ccsCOMPL_STAT lsfSoftDevConstructor

( IN  const char      *swdName,

IN  lsfCONTROL_DATA *controlData,

IN  const char      *softDevNames[],

OUT ccsERROR        *error )

This is the constructor of the softDev object. It affects all the

software devices given in the list <softDevNames[]> (NULL terminated).

void          lsfSoftDevDestructor

( IN  lsfCONTROL_DATA *controlData )

This method is the object destructor. All resources are released.

ccsCOMPL_STAT lsfSoftDevInit

ccsCOMPL_STAT lsfSoftDevStandby

ccsCOMPL_STAT lsfSoftDevOnline

ccsCOMPL_STAT lsfSoftDevStop

ccsCOMPL_STAT lsfSoftDevOff

ccsCOMPL_STAT lsfSoftDevExit

ccsCOMPL_STAT lsfSoftDevSimulat

ccsCOMPL_STAT lsfSoftDevStopsim

ccsCOMPL_STAT lsfSoftDevSelftest

ccsCOMPL_STAT lsfSoftDevTest ( IN  lsfSOFTDEV_DATA *softDevData,

IN  const char      *softDevName,

OUT ccsERROR        *error )

These methods forward the standard commands to the software

devices given ib the list <softDevName>.

If <softDevName> is NULL, or "all" all the software devices will

be affected, otherwise only the specified one.

It updates the state and subState of the group.

PRIVATE METHODS

static ccsCOMPL_STAT lsfSendCommand

( IN  const char *srvName,

IN  const char *command,

IN  const char *param,

OUT ccsERROR   *error )

This method sends the command <command> with the

parameters <param> to the server process <srvName> of the

software device.

static ccsCOMPL_STAT lsfWaitReply

( IN  const char *srvName,

IN  const char *command,

IN  char       *reply,

IN  int         timeout,

OUT ccsERROR   *error )

This method waits for the last or error reply to the command

<command> from the server process <srvName> of the software device.

The reply is stored in <reply>.

PRIVATE DATA MEMBERS

The data stucture lsfSOFTDEV_ENTRY is a sub-class of lsfDEVICE_ENTRY.

typedef struct

{

vltINT32         state;

vltINT32         subState;

vltLOGICAL       init;

vltLOGICAL       simulation;

vltINT32         timeout;

Additional data members

vltBYTES20  serverName;          Name of the server process

} lsfSOFTDEV_ENTRY;

The data stucture lsfSOFTDEV_DATA is a sub-class of lsfDEVICE_DATA.

typedef struct

{

void             *ctrlData;

void             *userData;

vltINT32          state;

vltINT32          subState;

vltLOGICAL        init;

vltLOGICAL        simulation;

vltINT32          timeout;

vltINT32          numSoftDevs;         Number of software devices

vltBYTES20       *softDevNames;        Names of the software devices

(dynamic allocated)

char             *attrTable[lsfMAX_DEVICES+1];

lsfSOFTDEV_ENTRY *softDevTable[lsfMAX_DEVICES];   SoftDev data

} lsfSOFTDEV_DATA;

DATABASE

The class lsfDB_SOFTDEV is a sub-class of lsfDB_DEVICE.

It is partially mapped by the data structure lsfSOFTDEV_ENTRY.

Each softDev is represented in the database by a point under the

main point :control:softDev, which is partially mapped by the data

structure lsfSOFTDEV_DATA.

SEE ALSO

lsfDB_DEVICE(5), lsfDB_SOFTDEV(5)

  - Database Classes

    - lsfTemplate.db

// *************************************************************************

// * E.S.O. - VLT project

// *

// * "@(#) $Id: lsfTemplate.db,v 1.27 2000/10/24 08:32:13 vltsccm Exp $"

// *

// * who       when        what

// * --------  ----------  ----------------------------------------------

// * pduhoux   2000-05-18  created

// *

// ************************************************************************

// *

// * This file has been generated by a utility

// *

// *  !!!!!!!!!!!  DO NOT MANUALLY EDIT THIS FILE  !!!!!!!!!!!

// *

// *----------------------------------------------------------------------

// */

//

// Points needed by the application 'pfx'

//

#include "pfxDefines.h"

#include "pfxDeviceList.h"

#include "lsfDB_SERVER.class"

#include "pfxDB_DEVICE.class"

#include "pfxDB_DATA.class"

#ifdef lsfNUM_DEVICES

#if lsfNUM_DEVICES > 0

#include "lsfDB_CONTROL.class"

CLASS "lsfDB_CONTROL" "pfxDB_CONTROL"

BEGIN

ATTRIBUTE Table deviceTable (lsfNUM_DEVICES)

#ifdef lsfNUM_SIGNAL

#if lsfNUM_SIGNAL > 0

ATTRIBUTE  "lsfDB_DEVICE"  lsfDEVTYPE_SIGNAL_STR

#endif

#endif

#ifdef lsfNUM_MOTOR

#if lsfNUM_MOTOR > 0

ATTRIBUTE  "lsfDB_DEVICE"  lsfDEVTYPE_MOTOR_STR

#endif

#endif

#ifdef lsfNUM_SERIAL

#if lsfNUM_SERIAL > 0

ATTRIBUTE  "lsfDB_DEVICE"  lsfDEVTYPE_SERIAL_STR

#endif

#endif

#ifdef lsfNUM_ENCODER

#if lsfNUM_ENCODER > 0

ATTRIBUTE  "lsfDB_DEVICE"  lsfDEVTYPE_ENCODER_STR

#endif

#endif

#ifdef lsfNUM_NETWORK

#if lsfNUM_NETWORK > 0

ATTRIBUTE  "lsfDB_DEVICE"  lsfDEVTYPE_NETWORK_STR

#endif

#endif

#ifdef lsfNUM_SOFTDEV

#if lsfNUM_SOFTDEV > 0

ATTRIBUTE  "lsfDB_DEVICE"  lsfDEVTYPE_SOFTDEV_STR

#endif

#endif

#ifdef lsfNUM_TASKDEV

#if lsfNUM_TASKDEV > 0

ATTRIBUTE  "lsfDB_DEVICE"  lsfDEVTYPE_TASKDEV_STR

#endif

#endif

END

#endif

#endif

CLASS "lsfDB_SERVER" "pfxDB_SERVER"

BEGIN

#ifdef lsfNUM_DEVICES

ATTRIBUTE int32     numDevices lsfNUM_DEVICES

#if lsfNUM_DEVICES > 0

ATTRIBUTE "pfxDB_CONTROL" control

BEGIN

ATTRIBUTE Table deviceTable (lsfNUM_DEVICES)

END

#endif

#else

ATTRIBUTE int32     numDevices 0

#endif

ATTRIBUTE "pfxDB_DATA" data

END

POINT "pfxDB_SERVER" pfxDB_ROOT

BEGIN

ALIAS pfxMODULE_NAME

//

// Instanciate all devices

//

#ifdef lsfNUM_DEVICES

#if lsfNUM_DEVICES > 0

//  ATTRIBUTE "<lsfDB_CLASS>"   control:<devType1>:<devName1>

//  ATTRIBUTE "<pfxDB_CLASS>"   control:<devType2>:<devName2>

#endif

#endif

END

// END OF FILE

// ============================================================

    - lsfDB_SERVER(5)

NAME

lsfDB_SERVER - Base class for software device application

SYNOPSIS

#include "lsfDB_SERVER.class"

POINT lsfDB_SERVER ":<myPath>:<myMod>"

BEGIN

ALIAS "myMod"

END

PARENT CLASS

lsfDB_DEVICE <-- lsfDB_SOFTDEV <-- lsfDB_SERVER

DESCRIPTION

The class lsfDB_SERVER is the generic definition of the top point of

a software device. It holds the minimum information required

by "lsf" for the software device management.

ATTRIBUTES

In addition to those defined in the parent class lsfDB_SOFTDEV:

monPeriod   : period in ms of the associated monitor task

numDevices  : number of controlled devices

    - lsfDB_CONTROL(5)

NAME

lsfDB_CONTROL - Base class for application control point

SYNOPSIS

#include "lsfDB_CONTROL.class"

ATTRIBUTE lsfDB_CONTROL control

PARENT CLASS

BASE_CLASS <-- lsfDB_CONTROL

DESCRIPTION

The class lsfDB_CONTROL is the generic definition of the

control point.

It contains the attribute <deviceTable> holding the information

relevant to the control of the devices.

The application control point is instanciated from a sub-class

of this generic class. In particular it overloads the number of

devices and adds a sub-point per device type.

ATTRIBUTES

- TABLE deviceTable(1)

deviceName : name of the device

deviceType : type of the device

startPhase : sequence order when increasing the device state

stopPhase  : sequence order when decreasing the device state

simulation : true if device shall be controlled in simulation

ignored    : true is device shall be ignored

CAUTIONS

The two fields <startPhase> and <stopPhase> are ignored.

    - lsfDB_DATA(5)

NAME

lsfDB_DATA - Base class for application data

SYNOPSIS

#include "lsfDB_DATA.class"

ATTRIBUTE lsfDB_DATA data

PARENT CLASS

BASE_CLASS <-- lsfDB_DATA

DESCRIPTION

This class is the generic definition of the data point.

It is provided to hold the application specific data used e.g.

for monitoring purpose.

The application data point is instanciated from a sub-class

of this generic class. Its definition is application dependent.

ATTRIBUTES

None

    - lsfDB_DEVICE(5)

NAME

lsfDB_DEVICE - Base class for devices

SYNOPSIS

#include "lsfDB_DEVICE.class"

ATTRIBUTE lsfDB_DEVICE  <aGenericDevice>

PARENT CLASS

BASE_CLASS <-- lsfDB_DEVICE

DESCRIPTION

The class lsfDB_DEVICE is the generic definition of a device.

It holds the minimum information required by "lsf" for the

device management. For each type of device a specialized

sub-class is provided that adds specific attributes.

ATTRIBUTES

lsfDB_DEVICE

state       : state of the device (= LCC states)

substate    : sub-state of the device

initialized : true if device is initialized

simulation  : true if device shall be controlled in simulation

timeout     : timeout in seconds for command execution

    - lsfDB_SIGNAL(5)

NAME

lsfDB_SIGNAL - Base classes for analogue and digital signals

SYNOPSIS

#include "lsfDB_SIGNAL.class"

ATTRIBUTE lsfDB_ANALOG  analog

ATTRIBUTE lsfDB_DIGITAL digital

PARENT CLASS

|-- lsfDB_ANALOG

lsfDB_DEVICE <-- lsfDB_SIGNAL <--+

|-- lsfDB_DIGITAL

DESCRIPTION

The class lsfDB_SIGNAL is the generic definition of a signal

(analogue or digital) device. It contains the attribute holding

the information relevant to the control of these devices.

ATTRIBUTES

The following attributes are mapping for Analog and Digital

signals the configuration parameters expexted by the LCC

functions ioConfigAnalog(3), resp. ioConfigDigital(3).

lsfDB_SIGNAL

In addition to those defined in the parent class lsfDB_DEVICE:

deviceName : name of the device "/acroN" for Digital signals

"/aioN"  for Analogue signals

direction  : signal direction (Input/Output)

lsfDB_ANALOG

deviceName : overloaded to "/aio"

In addition to those defined in the parent class lsfDB_SIGNAL:

channel    : channel number

gain       : amplifier gain

conversionFactor: floating point value

lowerRange : signal range

higherRange:

simValue   : simulation value

signalA    : associated database point (lccANALOG_SIGNAL)

lsfDB_DIGITAL

deviceName : overloaded to "/acro"

In addition to those defined in the parent class lsfDB_SIGNAL:

level      : active Low/High

startBit   : lower bit [0-63]

numBits    : number of bits mapping the signal

simValue   : simulation value

signalD    : associated database point (lccDIGITAL_SIGNAL)

SEE ALSO

LCC User Manual for Signal Handling

ioConfigAnalog(3), ioConfigDigital(3)

Digital I/O Board User Manual, and acro driver User Manual

Analog  I/O Board User Manual, and aio  driver User Manual

    - lsfDB_MOTOR(5)

NAME

lsfDB_MOTOR - Base class for motor

SYNOPSIS

#include "lsfDB_MOTOR.class"

ATTRIBUTE lsfDB_MOTOR <aMotorDevice>

PARENT CLASS

lsfDB_DEVICE <-- lsfDB_MOTOR

DESCRIPTION

The class lsfDB_MOTOR is the specialized class for motors of

any kind.

At instanciation, the class of the attribute <motor> shall be

overloaded with the sub-class corresponding to the motor.

ATTRIBUTES

In addition to those defined in the parent class lsfDB_DEVICE:

motor : motor branch (from motMOTOR)

SEE ALSO

motor.db(5), motMOTORS(5)

    - lsfDB_SERIAL(5)

NAME

lsfDB_SERIAL - Base classes for serial communication links

SYNOPSIS

ATTRIBUTE lsfDB_SERIAL com

ATTRIBUTE lsfDB_RS232  com1

ATTRIBUTE lsfDB_RS422  com2

ATTRIBUTE lsfDB_RS485  com3

PARENT CLASS

|-- lsfDB_RS232

lsfDB_DEVICE <-- lsfDB_SERIAL <--+-- lsfDB_RS422

|-- lsfDB_RS485

DESCRIPTION

The class lsfDB_SERIAL is the generic definition of a serial

communication device. It contains the attribute holding the

information relevant to the control of these devices.

The 3 sub-classes lsfDB_RS232, lsfDB_RS422 and lsfDB_RS485 are

specialized classes for the control of RS232, RS422 and RS485 resp.

communication links based on these protocols.

ATTRIBUTES

lsfDB_SERIAL

In addition to those defined in the parent class lsfDB_DEVICE:

deviceName  : name of the device "/iser0-9" or "/tyCo/1-3"

protocol    : number of the RS protocol [232, 422 or 485]

for the following attributes, see iserDrv(3) and VxWorks

manuals for tyCo devices

baudRate    :

dataBits    :

stopBits    :

parity      :

handShake   :

rxMode      :

bufferLength:

lsfDB_RS232, lsfDB_RS422 and lsfDB_RS485: these sub-classes overload

the attributes 'protocol' and 'handShake'.

SEE ALSO

iserDrv(3), VxWorks tyLib(1), ttyLib(1) and related

    - lsfDB_TASKDEV(5)

NAME

lsfDB_TASKDEV - Base classes for task devices

SYNOPSIS

#include "lsfDB_TASKDEV.class"

ATTRIBUTE lsfDB_TASKDEV <aTaskDevice>

PARENT CLASS

lsfDB_DEVICE <-- lsfDB_TASKDEV

DESCRIPTION

The class lsfDB_TASKDEV is the specialized class for tasks

devices to be controlled by the application.

ATTRIBUTES

lsfDB_TASKDEV

In addition to those defined in the parent class lsfDB_DEVICE:

startState  : minimum state for task activation

timerNum    : timer number for period

-1 : use system call taskDelay()

1 - 4 : use TIM timer

period      : task period in milli-seconds

priority    : task priority

stackSize   : task stack size in kilo-bytes [kB]

    - lsfDB_SOFTDEV(5)

NAME

lsfDB_SOFTDEV - Base classes for software devices

SYNOPSIS

#include "lsfDB_SOFTDEV.class"

ATTRIBUTE lsfDB_SOFTDEV <aSoftwareDevice>

PARENT CLASS

lsfDB_DEVICE <-- lsfDB_SOFTDEV

DESCRIPTION

The class lsfDB_SOFTDEV is the specialized class for Software

devices to be controlled by the application.

ATTRIBUTES

lsfDB_SOFTDEV

In addition to those defined in the parent class lsfDB_DEVICE:

serverName  : name of the software device server

  - Include Files

    - lsfDefines.h

#ifndef LSF_DEFINES_H

#define LSF_DEFINES_H

/*******************************************************************************

* E.S.O. - VLT project

*

* "@(#) $Id: lsfDefines.h,v 1.27 2000/10/24 08:32:16 vltsccm Exp $"

*

* who       when        what

* --------  ----------  ----------------------------------------------

* pduhoux   2000-04-10  created

*/

/************************************************************************

*

*----------------------------------------------------------------------

*/

/*

* Constants

*/

#define _eccs_tostr(a) #a

#define _eccs_tostr_pass2(a) _eccs_tostr(a)

#define __FILE_LINE__  __FILE__ ":" _eccs_tostr_pass2(__LINE__)

#define lsfMODULE_ID    "lsf"	/* module name */

#define lsfMODULE_NAME   lsfMODULE_ID

#define lsfMODULE_TITLE  "LCU Server Framework"

#define lsfLOG_ID 	 100

#define lsfMAX_DEVICES          32

/*

* Database

*/

#define lsfDB_CONTROL_POINT    ":control"

#define lsfDB_DATA_POINT       ":data"

#define lsfDB_SIGNAL_POINT     ":signal"

#define lsfDB_MOTOR_POINT      ":motor"

#define lsfDB_SERIAL_POINT     ":serial"

#define lsfDB_ENCODER_POINT    ":encoder"

#define lsfDB_NETWORK_POINT    ":network"

#define lsfDB_SOFTDEV_POINT    ":softdev"

#define lsfDB_TASKDEV_POINT    ":taskdev"

#define lsfALL_DEVICES         "all"

#define lsfDEVTYPE_SIGNAL_STR     "signal"

#define   lsfDEVTYPE_ANALOG_STR   "analog"

#define   lsfDEVTYPE_DIGITAL_STR  "digital"

#define lsfDEVTYPE_MOTOR_STR      "motor"

#define lsfDEVTYPE_SERIAL_STR     "serial"

#define   lsfDEVTYPE_TYCO_STR     "tyCo"

#define   lsfDEVTYPE_RS232_STR    "rs232"

#define   lsfDEVTYPE_RS422_STR    "rs422"

#define   lsfDEVTYPE_RS485_STR    "rs485"

#define lsfDEVTYPE_ENCODER_STR    "encoder"

#define   lsfDEVTYPE_IK320_STR    "ik320"

#define lsfDEVTYPE_NETWORK_STR    "network"

#define   lsfDEVTYPE_ETHERNET_STR "ethernet"

#define   lsfDEVTYPE_NET01_STR    "net01"

#define   lsfDEVTYPE_ATM_STR      "atm"

#define lsfDEVTYPE_SOFTDEV_STR    "softdev"

#define lsfDEVTYPE_TASKDEV_STR    "taskdev"

#define lsfDEVICE_NUM_TYPES 7

#define lsfDEVICE_SIGNAL    1

#define lsfDEVICE_MOTOR     2

#define lsfDEVICE_SERIAL    3

#define lsfDEVICE_ENCODER   4

#define lsfDEVICE_NETWORK   5

#define lsfDEVICE_SOFTDEV   6

#define lsfDEVICE_TASKDEV   7

/* lsfDEVTYPE_<device> = (0x100 << (lsfDEVICE_<type>-1)) + <kind> */

#define lsfDEVTYPE_MASK       0xFF00

#define lsfDEVTYPE_UNKNOWN    0

#define lsfDEVTYPE_SIGNAL     0x0100

#define   lsfDEVTYPE_ANALOG   0x0101      /* = lsfDEVTYPE_SIGNAL | ioANALOG */

#define   lsfDEVTYPE_DIGITAL  0x0102      /* = lsfDEVTYPE_SIGNAL | ioDIGITAL */

#define lsfDEVTYPE_MOTOR      0x0200

#define lsfDEVTYPE_SERIAL     0x0400

#define   lsfDEVTYPE_RS232    0x0401

#define   lsfDEVTYPE_RS422    0x0402

#define   lsfDEVTYPE_RS485    0x0404

#define lsfDEVTYPE_ENCODER    0x0800

#define   lsfDEVTYPE_IK320    0x0801

#define lsfDEVTYPE_NETWORK    0x1000

#define   lsfDEVTYPE_ETHERNET 0x1001

#define   lsfDEVTYPE_NET01    0x1002

#define   lsfDEVTYPE_ATM      0x1004

#define lsfDEVTYPE_SOFTDEV    0x2000

#define lsfDEVTYPE_TASKDEV    0x4000

#define lsfSTATE_OFF     1

#define lsfSTATE_LOADED  2

#define lsfSTATE_STANDBY 3

#define lsfSTATE_ONLINE  4

#define lsfUNCHANGED -99

#define lsfSUBSTATE_IDLE         0

#define lsfSUBSTATE_ERROR        1

#define lsfSUBSTATE_TIMEOUT      2

#define lsfSUBSTATE_INITIALIZING 3

#define lsfSUBSTATE_ACTIVE       4

#define lsfSUBSTATE_MONITORING   5

#define lsfSUBSTATE_MOVING       6

#define lsfSUBSTATE_WAITING      7

#define lsfATTR_NAMES \

"state","substate","simulation","initialized","timeout"

#define lsfDB_STATE          0

#define lsfDB_SUBSTATE       1

#define lsfDB_SIMULATION     2

#define lsfDB_INITIALIZED    3

#define lsfDB_TIMEOUT        4

#define lsfDB_LAST           5

#endif /*!LSF_DEFINES_H*/

<!DOCTYPE HTML PUBLIC "-//IETF//DTD HTML//EN">

- Installation Guide

This chapter is a guide for the installation of Software Devices in the VLT environments. It is built in 2 sections resp. for the WS and the LCU parts. The installation of the Software Device on the Workstation environment depends on the requirements, namely whether the database must be scanned from the LCU environment. In that case, the database shall contain the branch mirroring the LCU database branch for scanning.

It is assumed that the WS environment wsEnv  and LCU environment lcuEnv  have been successfully created and configured for lcuEnv  to report to wsEnv.

    - WS Environment

The configuration of the WS environment is made in 3 steps:

- Edit the file $VLTDATA/ENVIRONMENTS/wsEnv/dbl/DATABASE.db
Add the following 2 lines outside any BEGIN ... END block:

#define appDB_ROOT "<absolute path>"

#include "app.db"

- Edit the file $VLTDATA/ENVIRONMENTS/wsEnv/dbl/Makefile
Add the following directive:

USER_INC = -I${INTROOT}/vw/include

- Generate the database, initiatialize and start the environment:

> cd $VLTDATA/ENVIRONMENTS/wsEnv/dbl

> make db

> vccEnvInit -e wsEnv

> vccEnvStart -e wsEnv

>

    - LCU Environment

The configuration of the LCU environment is made in 4 steps:

- Edit the file $VLTDATA/ENVIRONMENTS/lcuEnv/dbl/DATABASE.db
Add the following 2 lines outside any BEGIN ... END block:

#define appDB_ROOT "<absolute path>"

#include "app.db"

- Generate the database:

> cd $VLTDATA/ENVIRONMENTS/lcuEnv/dbl

> make db

>

- Edit the file $VLTDATA/ENVIRONMENTS/lcuEnv/devicesFile

- Increment the number of devices in the line:

- <ATTRIBUTE>: deviceTable <TYPE>: Table <REC>: 0 - N <FIELDS>: 0 - 5

- where N  must be incremented

- Add the following entry:

- "app"  "appServer" 1 0 0 1

IMPORTANT NOTE: Any Software Device that is to be controlled from another Software Device must not be registered in this list; the respect of this rule is essential for the system to behave correctly: since the standard commands are forwarded from the co-ordinating Software Device to the sub-ordinated ones, any standard command sent to LCC (process lccServer) would be forwarded to all the registered Software Devices found in this list, but as well from the co-ordinating Software Devices to the sub-ordinated ones resulting in non predictable effects.

- Configure the LCU bootScript and reboot the LCU:

> vccConfigLcu lcuEnv &

>

In the LCU Configuration panel:

- Update the user module list that shall contain:
lsf and app in this precedence order.
For applications that do use motors, the module mcm must be added.

- Add the required driver devices,

- Add the process appServer.

- Click on the button Write Files to save the configuration

- Click on the button Reboot LCU to boot the LCU

    - Scan Links

After both environments have been successfully started, the scan links can be configured. Edit the file app/ws/config/app.scan to add all the links required by the application.

Example:

> cd app/ws/config

> vi app.scan

...

> cd ../src

> make install

...

. . . installation done

>

> scanLinks -f $INTROOT/config/app.scan -l wsEnv -e lcuEnv -c -E -r <path>

>

where <path> is the absolute path of the database root point of the application on the WS.

The WS database will now be kept aligned with the LCU database by means of the scan system. In particular, the module state and sub-state are updated on change (mode SRBX).

    - Verification

On the Workstation, invoke the panel appgui.

The fields State, Substate and Mode shall not be shaded (indication for bad data quality, hence scan troubles). They shall indicate LOADED (in Orange), IDLE (in Green) and NORMAL (in Green) resp.

Send the command VERSION.

The Software Device is ready for use, however it might be necessary to tune the configuration of the devices (e.g. motors or signals).

    - Database configuration

This last section is dedicated to the handling of the database configuration file of the application (app/ws/config/app.dbcfg).

As mentioned above, this file is generated by the utility lsfConfig(1). It contains then the minimum necessary configuration attributes for the software device to startup. After the tuning of the hardware has been completed, i.e. all devices are properly configured and initialize and perform as required, the database shall be saved into the file app/ws/config/app.dbcfg by means of the utility lsfBackup(1).

Example:

> cd app/ws/config

> lsfBackup -e lcuEnv -m app

Generating input file './app.inp' ... done

Performing database backup into 'app.dbcfg' ... done

> cd ../src

> make install

...

. . . installation done

>

<!-- Standard footer -->

<!-- hhmts start -->Last modified: Thu Sep 28 10:32:15 METDST 2000 <!-- hhmts end -->

___oOo___